TECHNICAL CHARACTERISTICS

GSM / GPRS FEATURES

- E-GSM 900/1800 MHz (or optional 850/1900MHz)

- ETSI GSM Phase 2+

- Class 4 (2W @ 900 MHz)

- Class 1 (1W @ 1800 MHz)

- SIM Toolkit Release 99

VOICE FEATURES

- Voice (GSM mode)

- Telephony, Emergency Numbers 112

- Full Rate, Enhanced Full Rate, Half Rate

(FR/EFR/HF)

- Echo cancellation and noise reduction

- Full Duplex Hands Free

DATA FEATURES

- GPRS Class 10 (Up to 4Rx / 2Tx)

- Supports PBCCH, coding schemes: CS1 to CS4

- Asynchronous data circuit, transparent and non

transparent up to 14400 bits/s

- SMS point to point MT/MO and SMS CB

GPS FEATURES

- 16 channel μblox receiver

- Precision:

• 2,5 metres CEP

• GPS 2m CEP

• A-GPS Compatible

- Acquisition time:

• Hot start: < 3,5 sec

• Cold start: 36 sec

- Re-acquisition time after loss of signal < 1 s

- High sensitivity: -158 dBM when Tracking

- Protocols:

• NMEA-0183 Input/output

• UBX binary input/output

• RTCM in

- Integrated supply for active 3V GPS antenna

- Antenna fault detection (absence or short-circuit)

INTERFACES

- Antenna, 2 separate connectors:

• GSM: SMA-F

• GPS: SMB-M

- Power supply: 5.5Vdc to 32Vdc (micro-FIT connector)

Reklamni Inženiring d.o.o.

Dunajska 116

1000 Ljubljana

Slovenia

EXPLORER

- RS232 + Audio via a female 15-pin Sub-D

- AT commands: GSM 07.05 and 07.07

- Specific EXPLORER AT commands:

• Multiplexage DSR / position (NMEA)

• Multiplexage DTR / position (NMEA)

- 3 opto-coupled inputs, 3Vdc to 32Vdc (micro-FIT

connector)

- 1 open-collector output 60Vdc - 1A (micro-FIT

connector)

- SIM card reader (SIM 3V - 1,8V)

- Buzzer output

- Power and I/O cables included

AVERAGE POWER CONSUMPTION

- GSM 900 MHz: 105 mA @ 12 V in communication

- GSM 1800 MHz: 80 mA @ 12 V in communication

- Idle mode: 5 mA @ 12 V (GPS off)

- GPS: 38 mA @ 12 V

ENVIRONMENTAL & MECHANICAL

- Operating temperature: - 20°C to + 55°C

- Storage temperature: - 30°C to + 85°C

- aluminium case:

Length 73 mm x Width 54 mm x Depth 25 mm

- Weight: 95g

APPROVALS

- R&TTE (Radio & Telecom. Terminal Equipment)

- Automobile approval: E10R-020250 ("E" Label)

OPTIONS

- Flash memory extension

- Shock detector

- 850/1900MHz

- TCP/IP Library (PPPRFC, TCP Socket, UDP Socket,

SMTP, POP3, FTP)

- Approved magnetic μblox GPS antenna (SMB-F) (ref.:

60302B1501)

- GSM antenna (SMA-M)

- Serial Data cable - 15M/9F

or Data/Audio cable - 15M/9F/RJ9

- 230 Vac - 12 Vdc power supply

- EXPLORER Blue 10e:

remote-supply Bluetooth® / RS232 adapter

tel: 080 17 01

tel: +386 1 580 97 40

fax: +386 1 580 97 46

e-mail: info@explorer.si