- FALCOM A1

- User Manual / Command List

Contents

1.	Introduction	5

1.1.	Scope of this document	5

1.2.	Related documents	5

1.3.	Adresses	5

1.4.	Definitions	6

1.5.	Technical Data	6

2.	AT commands features	10

2.1.	FALCOM A1 line settings	10

2.2.	Command line	10

2.3.	Information responses and result codes	10

3.	AT commands for Call Control	11

3.1.	Dialing command  D	11

3.2.	Hang-Up command H	12

3.3.	Answer a call  A	12

3.4.	Remote disconnection	12

3.5.	DTMF signals  +VTD, +VTS	13

3.6.	Operator selection +COPS	14

3.7.	Redial last number ATDL	14

3.8.	Signal Quality  +CSQ	15

3.9.	Network registration +CREG	15

3.10.	Automatic answer  ATS0	15

3.11.	Audio gain control   +VGR, +VGT	16

3.12.	SideTone modification +SIDET	17

3.13.	Echo canceler   +ECHO	17

4.	AT commands SIM, E2prom	19

4.1.	Product Serial Number +CGSN	19

4.2.	Request model identification +CGMM	19

4.3.	Request revision identification +CGMR	19

4.4.	Manufacturer identification +CGMI	20

4.5.	Capabilities list  +GCAP	20

4.6.	Phone activity status  +CPAS	20

4.7.	Power off  +CPOF	20

4.8.	Set phone functionality +CFUN	21

4.9.	Enter PIN/PUK code +CPIN	21

4.9.1.	Remark	23

4.10.	Facility lock  +CLCK	23

4.11.	Change password  +CPWD	24

4.12.	Select phonebook memory storage +CPBS	25

4.13.	Read phonebook entries +CPBR	25

4.14.	Find phonebook entries +CPBF	26

4.15.	Write phonebook entiy +CPBW	27

4.16.	Advice of charge +CAOC	27

4.17.	Accumulated call meter +CACM	28

4.18.	Accumulated call meter maximum +CAMM	29

4.19.	Price per unit and currency table +CPUC	29

5.	AT commands for short messages	30

5.1.	Parameters definition	30

5.2.	Select message service  +CSMS	31

5.3.	Preferred Message Storage +CPMS	31

5.4.	Preferred Message Format +CMGF	31

5.5.	Save settings +CSAS	32

5.6.	Restore settings +CRES	33

5.7.	Show text Mode Parameters +CSDH	33

5.8.	Select TE character set +CSCS	33

5.9.	New message indication  +CNMI	34

5.10.	Read message  +CMGR	36

5.11.	List message +CMGL	37

5.12.	Send message  +CMGS	38

5.13.	Set Text Mode Parameters  +CSMP	38

5.14.	Delete message  +CMGD	39

5.15.	Service center address +CSCA	39

6.	AT commands for Supplementary Services	40

6.1.	Call forwarding  +CCFC	40

6.2.	Call barring +CLCK	40

6.3.	Modify SS password  +CPWD	41

6.4.	Call waiting +CCWA	41

6.5.	Calling line identification restriction +CLIR	42

6.6.	Calling line identification presentation +CLIP	43

6.7.	Connected line identification presentation  +COLP	43

7.	AT commands for data	45

7.1.	Bearer type selection +CBST	45

7.2.	Report control  +CR, +CRC	46

7.2.1.	Outgoing call +CR	46

7.2.2.	Incoming call +CRC	46

7.3.	Echo   E	47

7.4.	Fixed DTE rate   +IPR	47

7.5.	Back to online mode  O	48

7.6.	DCE response format V	48

7.7.	DTE-DCE character framing +ICF	48

7.8.	Repeat last command  A/	49

7.9.	Default configuration  Z	49

7.10.	DTE-DCE local flow control +IFC	49

7.11.	Set DCD signal  &C	50

7.12.	Set DTR signal  &D	50

7.13.	Set DSR signal  &S	50

7.14.	Save configuration  &W	51

7.15.	Radio link protocol parameters +CRLP	51

8.	Other AT commands	52

8.1.	V.25 ter recommendation	52

8.2.	GSM 07.05 recommendation	52

8.3.	GSM 07.07 recommendation	52

9.	Failure result codes	53

9.1.	Report Mobile Equipement errors +CMEE	53

9.2.	Extended error report +CEER	53

9.3.	Mobile equipment error result code : +CME ERROR: xxx	54

9.4.	Message service failure result code:  +CMS ERROR :<er>	54

9.5.	Cause information element values from GSM recommendation 04.08	55

9.6.	GSM 04.11 Annex E-2:  Mobile originating SM-transfer	58

10.	Fax management	61

10.1.	Setting up the module for fax:	61

10.2.	Setting up the PC fax application:	61

10.2.1.	Delrina WinFax v4.0	61

10.3.	Restoring the module default profile for data call	62

11.	General	63

11.1.	GENERAL SAFETY	65

11.2.	SAFETY STANDARDS	66

# Introduction

## Scope of this document

This document describes the messages exchanged between an external application module and the FALCOM A1 GSM mobile station, based on AT commands in order to control incoming and outgoing calls.

This document presents the AT-commands implemented for the voice call tele-service, the data tele-service and the short message teleservice.

## Related documents

This interface specification is based on the following recommendations:

- ETSI GSM 07.05: European digital cellular telecommunication system (phase 2); Use of DTE-DCE interface for Short message service and cell broadcast service

- ETSI GSM 07.07: European digital cellular telecommunication system (phase 2); AT command set for GSM Mobile Equipment

- ITU-T Recommandation V.25 ter: Serial asynchronous automatic dialling and control

- ETSI GSM 03.38: European digital cellular telecommunication system (phase 2); Alphabets and language-specific information.

## Adresses

European Telecommunications Standards Institute

Secretariat

650 Route des Lucioles

F-06921 Sophia Antipolis – Valbonne

X.400: c=fr, a=atlas, p=etsi, s=secretariat

Tel: (+33) 92 94 42 00 Fax (+33) 93 65 47 16

Email: secretariat@etsi.fr

http://www.etsi.fr

## Definitions

In the GSM vocabulary, a call from GSM mobile to the Network is named “mobile originated call“ or „outgoing call“, and a call from fixed network to a GSM mobile is called „mobile terminated call“ or „incoming call“.

## Technical Data

- Dimensions:	B x H x T	115mm x 54mm x 33mm

- Weight:		130g

- Power Supply:	10,8...31,2 V/DC (worst case, including application board on basis

- DS 87C520/PLCC44)

325 mA transmit mode 10,8V

47 mA idle mode 10,8V

110 mA transmit mode 31,2V

25 mA idle mode 31,2V

- Temperature Range:	Storage: -20°C to +70°C

Use:       -20°C to +55°C

- Interface A:	Antenna  50 FME Male

- Interface B:	RS232 / V24   9 pin D-Sub  to DIN 41652

This interface is the master and is connected to the GSM-module, when RTS is active. Under this status the LED line is swiched off, only power on is displayed.

- Interface C:	RJ 45   8 pin shielded for hand set, RX, TX, Power and ground

- Interface D:	Power supply

OPEN CABLE:	white	+10,8 - 31,2V

yellow	ignition (+)  (option)

brown	ground

- SIM-Card interface:	for small SIM-Cards

- Digital interface:	V24 (9 pole D-Sub)

- V28

AT-commands according to ETSI GSM 0707 / 05

Extended AT-commands List

- Data communication:	asynchronous, transparent and not transparent

- GSM 07.01, 07.02, 04.21

- 2400 BPS V22 bis

- 2400 BPS V26 ter

- 4800 BPS V32

- 9600 BPS V32

- 2400 BPS V110

- 4800 BPS V110

- 9600 BPS V110

- Routes:			MOC and MTC

INFO transf. mode:	3,1Khz audio ex PLMN (analogue) or UDI/V.110 (ISDN)

Clock mode:		Asynchronous

Connection elem.:		Transparent/Non Transparent

Structure:			Unstructured/Structured

Layer 2 protocol:		NAV

Data bits:			8

Parity bits:			None

Bits/char.:			Always 10 bits/char.

Stop bits:			1

Radio channel:		Full rate

Intermed. rate:		16 kbit/s

User rate:			2400, 4800 or 9600 bit/s

Modem type:		V.22bis, V.26ter, Auto-baud, V.32

Bearer service:		BS 24, BS 25, BS 26

- Short Message Service: GSM 03,40, 07.05

- SMS mobile originated

- SMS mobile terminated

- CBS	Text mode /PDM mode

7 bit user data / 8 bit user data

- Routes:		SMS MO and MT

MODE:		Text mode, PDU, 7/8 bits user data

Teleservice:	TS22, TS21

- Fax:

- Group 3 /Class I

CCITT Rec.:			CCITT T.30, T.4

Routes:			MOC and MTC

Info. transf. mode:		Synchronous

Connection elem.:		Transparent

Structure:			Unstructured

Radio channel:		Full rate

Intermed. rate:		16kbit/s

User rate:			2400,4800 or 9600 bit/s

Teleservice:			TS 62

- Audio interface:

- Electret-Microphone

- loudspeaker 150

- Power 6V regulated DC

- ground

- shield

- LED interface: 	The actual status of the GSM-modul FALCOM A1 is

displayed by a line of 8 LED

- LED green		Power on

- LED red		not ready / no GSM-net

In case of LED red is dark, the set is booked in the GSM-net.

- LED orange	fieldstrenth of the GSM-net

# AT commands features

## FALCOM A1 line settings

Currently, FUNKANLAGEN LEIPOLDT OHG's handler is set to 9600 bps, 8 bits data, 1 bit stop, no parity, RTS/CTS flow control. Please use the commands +IPR, +IFC, +ICF to change these settings.

## Command line

Commands always finish with <CR>.

## Information responses and result codes

Responses always start with <CR><LF> and end with <CR><LF>.

- If the syntax command is wrong, the string ERROR is sent.

- If the syntax command is correct but with some incorrect parameters, the string +CME ERROR: <err> is sent with different error codes.

- If the command line has been performed successfully, the string OK is sent.

Please note that in the some cases like "AT+CPIN?", "AT+EXPKEY?", incoming events etc... the mobile does not finish its response with the string OK.

In the following examples <CR> and <CR><LF> will be intentionally omitted.

# AT commands for Call Control

## Dialing command		D

This AT command is used by the external application to establish a call.

For a data call, the application sends to the GSM module the following ASCII string : (the bearer has to be selected before with the +CBST command)

ATD<nb>;	where <nb> is the called phone number.

For a voice call, the application sends to the GSM module the following ASCII string : (the bearer may be selected before, if not a default bearer is used)

ATD<nb>;	where <nb> is the called phone number.

The answer to the command is the following :

| Verbose result code | Numeric (V0 set) | Desciption |
| --- | --- | --- |
| OK | 0 | if the call succeeds, for voice call only |
| CONNECT <speed> | 10,11,12,13,14,15 | the call succeeds, for data call only, <speed> takes the value negotiated by the GSM module |
| BUSY | 7 | if the called party is in communication |
| NO ANSWER | 8 | if no hang up is detected after a fixed network time-out |

Please, notice that in case of international number, the local international prefix has not to be set (usually 00) but need to be replaced by the '+' character.

Example : to establish a voice call to FUNKANLAGEN LEIPOLDT OHG from another country, the AT command shall be :

- ATD+33146290800

Notice that some country can have particular numbering rules for their GSM handset numbering.

Direct dialling from a phonebock location (stored in SIM card) be done with the following command:

ATD>17;		or		AD>SIM17;		(voice call here)

ATD>”Fred”;	for calling “Fred” from the phonebook

## Hang-Up command	H

This command is used by the application to disconnect the remote user. The application sends :

| Application to GSM | ATH | ask for disconnection |
| --- | --- | --- |
| GSM to application | OK | even if there is no communication established |

## Answer a call		A

When the GSM module receives a call, it set the RingInd signal and sends to the application the ASCII string RING. Then it waits for the application to accept the call.

| Application to GSM | ATA |  |
| --- | --- | --- |
| GSM to application | OK | Normal case |
| Application to GSM | ATA |  |
| GSM to application | NO CARRIER | if ATA is sent when no incoming call was indicated (operation not allowed) |

## Remote disconnection

This message is used by the GSM module to indicate to the application that the communication has been released by the remote user.

The GSM module sends  "NO CARRIER"  to the application, and set the DCD signal.

## DTMF signals		+VTD, +VTS

The GSM module offers the user application to send DTMF tones. One command shall be used for defining the duration of the tones (default value is 70 ms), the other for sending the Tones.

For defining the duration, the application uses :

AT+VTD=<n>		where <n>*100 gives the duration in ms.

| Application to GSM | AT+VTD=1 | for defining 100 ms tone duration |
| --- | --- | --- |
| GSM to application | OK | Command valid |
| Application to GSM | AT+VTD=100000 |  |
| GSM to application | +CME ERROR: 3 | if the duration is too long (the limit is to define for each application) |
| Application to GSM | AT+VTD=0 | for setting the manufacturer default value |
| GSM to application | OK |  |

For sending DTMF tones, the application uses :

AT+VTS=<Tone>	where <Tone> is in {0-9,*,#,A,B,C,D}

| Application to GSM | AT+VTS=A |  |
| --- | --- | --- |
| GSM to application | OK | Command valid |
| Application to GSM | AT+VTS=11 |  |
| GSM to application | +CME ERROR: 4 | If the <Tone> is wrong |
| Application to GSM | AT+VTS=4 |  |
| GSM to application | +CME ERROR: 3 | If there is no communication |

Example : to send the Tone sequence 13#, the application shall send :

AT+VTS=1			and then wait for	
OK
AT+VTS=3			and then wait for	
OK
AT+VTS=#			and then wait for	
OK

## Operator selection	+COPS

To select the operator, two possibilities exist :

The mobile enters the manual mode, and then try to find an operator which is indicated by the application. If it finds and registers correctly, then the mobile stays in idle mode.

The mobile enters the automatic mode, and then try to find the home operator. If it finds and registers correctly, then the mobile stays in idle mode ; if not, the mobile looks itself automatically for another network.

To force an attempt to select and register a GSM network operator, the application must send the following command :

- AT+COPS=<mode>, <format>, <oper>

The first parameter indicates either automatic (value 0) or manual (value 1). The second parameter indicates the format of the third parameter. The only format supported by FUNKANLAGEN LEIPOLDT OHG module is the numerical format (value 2). The third parameter is the operator identifier (MCC3 MCC2 MCC1 MNC2 MNC1).

| Application to GSM | AT+COPS? | ask for current plmn |
| --- | --- | --- |
| GSM to application | +COPS: 0,2,20801 OK | Home plmn is France Telecom |
| Application to GSM | AT+COPS=? | ask for plmn list |
| GSM to application | +COPS: (2,20801), (0,20810) OK | Home plmn is France Telecom SFR network has been seen |
| Application to GSM | AT+COPS=1,2,20810 | ask for registration on SFR network |
| GSM to application | +CME ERROR: 3 | failed |
| Application to GSM | AT+COPS=1,1,20810 | ask for registration on SFR network |
| GSM to application | +CME ERROR: 4 | wrong parameters |
| Application to GSM | AT+COPS=0,2 | ask for registration on home network |
| GSM to application | OK | succeeded |

## Redial last number	ATDL

This command is used by the application to redial the last number used in the command ATD. The application sends :

| Application to GSM | ATDL | redial last number |
| --- | --- | --- |
| GSM to application | 0146290800 OK | last call was a speech call command valid |

## Signal Quality		+CSQ

This command shall be used to know the signal strength indication.

| Application to GSM | AT+CSQ |  |
| --- | --- | --- |
| GSM to application | +CSQ: <rssi>,<ber> OK | <rssi> and <ber> as defined in the GSM TS 07 07 |

## Network registration	+CREG

This command is used by the application to know the registration status of the mobile.

<mode>

0: Disable network registration unsolicited result code

1: Enable network registration code result code +CREG: <stat>

<stat>

0: not registered, ME is not currently searching a new operator

1: registered, home network

2: not registered, ME currently searching a new operator to register to

3: registration denied

4: unknown

5: registered, roaming

| Application to GSM | AT+CREG? |  |
| --- | --- | --- |
| GSM to application | +CREG: <mode>,<stat> OK | as defined in the GSM TS 07 07 |
| Application to GSM | AT+CREG=<mode> | Disable/enable network registration unsolicited result code |
| GSM to application | OK | command valid |

## Automatic answer		ATS0

This S-parameter controls the automatic answering feature of the mobile.

| Application to GSM | ATS0=2 | automatic answer after 2 rings |
| --- | --- | --- |
| GSM to application | OK |  |
| Application to GSM | ATS0? | Current value |
| GSM to application | 002 OK | always 3 characters with leading zeros |
| Application to GSM | ATS0=0 | no automatic answer |
| GSM to application | OK | command valid |

## Audio gain control			+VGR, +VGT

This command shall be used by the application to tune the receive gain of the speaker and transmit gain of the microphone. The application shall send

| AT+VGT=<val> | for the receive gain | AT+VGR=<val> | for the transmit gain |
| --- | --- | --- | --- |
| 0 to 15 | +6 db | 0 to 31 | +14 db |
| 16 to 31 | +4 db | 32 to 63 | +17 db |
| 32 to 47 | +2 db | 64 to 95 | +20 db |
| 48 to 63 | 0 db | 96 to 127 | +23 db |
| 64 to 79 | -2 db | 128 to 159 | +26 db |
| 80 to 95 | -4 db | 160 to 191 | +29 db |
| 96 to 111 | -6 db | 192 to 223 | +32 db |
| 112 to 127 | -8 db | 224 to 255 | +35 db |
| 128 to 143 | -10db |  |  |
| 144 to 159 | -12 db |  |  |
| 160 to 175 | -14 db |  |  |
| 176 to 191 | -16 db |  |  |
| 192 to 207 | -18 db |  |  |
| 208 to 223 | -20db |  |  |
| 224 to 239 | -22 db |  |  |
| 240 to 255 | -24 db |  |  |

16 levels for receive gain are provided and 8 levels for transmit gain.

| Application to GSM | AT+VGR=25 |  |
| --- | --- | --- |
| Application to GSM | AT+VGR=25 |  |
| GSM to application | OK | Command valid |
| Application to GSM | AT+VGT=45 |  |
| GSM to application | OK | Command valid |

## SideTone modification	+SIDET

The syntax of the command is:

AT+SIDET=<val1>. <val2>

This commabd us used to get an echo of the voice in the speaker (to hear what is said in the speaker).

| <val2> | level | <val1> | Presence |
| --- | --- | --- | --- |
| 0 | 0 db | 0 | SideTone is disabled |
| 1 | - 6db | 1 | SideTone is enabled |
| 2 | - 12db |  |  |
| 3 | - 18 db |  |  |

| Application to GSM | AT+SIDET=1,0 |  |
| --- | --- | --- |
| GSM to application | OK | Command valid |
| Application to GSM | AT+SIDET? | Current value |
| GSM to application | +SIDET: 1,0 OK | Command valid |

## Echo canceler 		+ECHO

The syntax of the command is:

AT+ECHO=<val1>,<val2>

This command is used for car kit applications to reduce the echo effects than can occur in a car.

| <val2> | attenation | <val1> | Presence |
| --- | --- | --- | --- |
| 0 | - 18db | 0 | Echo is disabled |
| 1 | - 24db | 1 | Echo is enabled |
| 2 | - 30 db |  |  |
| 3 | - 36db |  |  |

| Application to GSM | AT+ECHO=1,0 |  |
| --- | --- | --- |
| GSM to application | OK | Command valid |
| Application to GSM | AT+ECHO? | Current value |
| GSM to application | +ECHO: 1,0 OK | Command valid |

# AT commands SIM, E2prom

## Product Serial Number	+CGSN

Product Serial Number	+CGSN

This command allows the user application to know the IMEI of the GSM module. The application sends :

| Application to GSM | AT+CGSN | get IMEI |
| --- | --- | --- |
| GSM to application | 135790248939 OK | IMEI present in E2PROM |
| Application to GSM | AT+CGSN | get IMEI |
| GSM to application | +CME ERROR: 22 | IMEI not found in E2PROM |

## Request model identification	+CGMM

This command is used to get the hardware version.

| Application to GSM | AT+CGMM | get hardware version |
| --- | --- | --- |
| GSM to application | GSM P 900 OK | command valid |

## Request revision identification	+CGMR

This command is used to get the software version.

| Application to GSM | AT+CGMR | get software version |
| --- | --- | --- |
| GSM to application | 276 OK | command valid |

## Manufacturer identification	+CGMI

This command gives the manufacturer identification.

| Application to GSM | AT+CGMI | get manufacturer identification |
| --- | --- | --- |
| GSM to application | FUNKANLAGEN LEIPOLDT OHG OK | command valid |

## Capabilities list		+GCAP

This command gives the complete capabilities list.

| Application to GSM | AT+GCAP | get capabilities list |
| --- | --- | --- |
| GSM to application | +CGSM OK | supports GSM commands and FAX |

## Phone activity status		+CPAS

This command returns the activity status of the mobile.

| Application to GSM | AT+CPAS | Current activity status |
| --- | --- | --- |
| GSM to application | +CPAS: "x" OK | <pas> can have the following values 0: ready (allow commands from TA/TE) 1: unavailable (does not allow cmds) 2: unknown 3: ringing (ringer is active) 4: call in progress 5: asleep (low functionality) |

## Power off		+CPOF

This command stops the GSM software stack and then the hardware layer.

| Application to GSM | AT+CPOF |  |
| --- | --- | --- |
| GSM to application | OK | command valid |

## Set phone functionality	+CFUN

This command selects the level of functionality in the mobile station.

- When the application wants to stop the module for making a power off, or if the application wants to force the module for executing a IMSI DETACH procedure, then it has to send :

- AT+CFUN=0

This command executes a IMSI DETACH and makes a backup of some internal parameters in the SIM and the EEPROM.

If the mobile is not powered off after this command, it shall received another command to re-start the whole GSM process.

If the mobile is turned off after this command, then the power on will automatically execute the start of the whole GSM process.

- When the application wants to re-start the module (after having done a AT+CFUN=0 command, and without having cut the power supply), it has to send :

- AT+CFUN=1,1

This command starts again all the GSM stack and GSM functionality.

| Application to GSM | AT+CFUN? | ask for current functionality |
| --- | --- | --- |
| GSM to application | +CFUN: "x" OK | "x" as defined in the GSM TS 07 07 |
| Application to GSM | AT+CFUN=0 | set minimum functionality |
| GSM to application | OK | command valid |
| Application to GSM | AT+CFUN=1 | set full functionality |
| GSM to application | OK | command valid |
| Application to GSM | AT+CFUN=1,0 | set full functionality without reset |
| GSM to application | OK | command valid |
| Application to GSM | AT+CFUN=1,1 | set full functionality with reset |
| GSM to application | OK | command valid |

## Enter PIN/PUK code	+CPIN

This command is used to validate the PIN code, or to validate the PUK code (and define a new PIN code).

If the user application try to establish an outgoing call before having validated the SIM PIN code, then the GSM module will refuse the "ATD" command with a "+CME ERROR: 11" (SIM PIN required).

It's up to the application to validate the PIN after each reset or power on. The application shall therefore use the command :

- AT+CPIN="<pin>"

| Application to GSM | AT+CPIN=1234 | enter PIN |
| --- | --- | --- |
| GSM to application | OK | PIN correct |
| Application to GSM | AT+CPIN=5678 | enter PIN |
| GSM to application | +CME ERROR: 16 | incorrect password |

After 3 unsuccessful codes, the PUK will then be required. The PUK validation forces the user to enter as a second parameter a new PIN code which will be the future PIN code if the PUK validation succeeds. The application shall therefore use the command :

- AT+CPIN="<puk>","<new pin>"

| Application to GSM | AT+CPIN=1234,5896 | enter PUK and new PIN |
| --- | --- | --- |
| GSM to application | OK | PUK correct |
| Application to GSM | AT+CPIN=5000,5896 | enter PUK and new PIN |
| GSM to application | +CME ERROR: 16 | incorrect password |

To know which code has to be entered (or not), the following interrogation command will be implemented :

- AT+CPIN?

The possible responses are :

| +CPIN: READY | No PIN is needed |
| --- | --- |
| +CPIN: SIM PIN | PIN is required |
| +CPIN: SIM PUK | PUK is required |
| +CME ERROR: <err> | SIM failure, SIM absent etc. |

Please note that in this case the mobile does not finish its response with the OK string.

- Example : 3 failed PIN validations + 1 successful PUK validation

| AT+CPIN? +CPIN: SIM PIN | The application asks for PIN status The GSM module requires PIN |
| --- | --- |
| AT+CPIN="1235" +CME ERROR: 16 | The application enters the PIN Bad PIN |
| AT+CPIN="1236" +CME ERROR: 16 | Second attempt bad PIN |
| AT+CPIN="1238" +CME ERROR: 16 | last attempt bad PIN |
| AT+CPIN? +CPIN: SIM PUK | and now ? the GSM module requires PUK |
| AT+CPIN="99999999","5678" OK | the PUK is entered, the new PIN shall be 5678 PUK validation is OK. New Pin is 5678 |
| AT+CPIN? +CPIN: READY | and now ? GSM module is ready |

### Remark

In the way Application to GSM, an "h" character shall be added before the PIN value if ciphering mode (with D.E.S algorithm ) is on. See +EXPKEY command.

Same remark for +CLCK and +CPWD commands.

## Facility lock		+CLCK

The syntax of this command is: +CLCK=<fac>,<mode>[<passwd>]

This command shall be used by the application to change a facility status,

with the following <fac> values:

“PS” : SIM lock facility with a 8 digits password.

“SC” : PIN enable (<mode>=1)/disable (<mode>=0)

“AO” : BAOC (Barr All Outgoing Calls)

“OI” : BOIC (Barr Outgoing International Calls)

“OX” : BOIC-exHC (Barr Outg. Intern. Calls exept to Home Country)

“AI” : BAIC (Barr All Incoming Calls)

“IR” : BIC-Roam (Barr Inc. When Roaming outside Home Country)

“AB” : All Barring services (only with <mode>=0)

“AG” : All outGoing barring services (only with <mode>=0)

“AC” : All inComing barring services (only with <mode>=0)

“WNL” : Network lock with a 8 digits password (NCK)

<mode>

0 : unlock the facility

1 : lock the facility

The password length is given with the AT+CPWD=? command.

| Application to GSM | AT+CLCK="SC",1,1234 | enable PIN |
| --- | --- | --- |
| GSM to application | OK | PIN iwas good |
| Application to GSM | AT+CLCK? | Read PIN status |
| GSM to application | +CLCK: ("PS", 0), ("SC", 1), ("AO", 0), ("OI", 0), ("OX", 0), ("AI", 0), ("IR", 0), ("AB", 0), ("AG", 0), ("AC", 0), ("WNL", 0) OK | PIN is enabled, all other facilities are disabled |
| Application to GSM | AT+CLCK="SC",0,5555 | disable PIN |
| GSM to application | +CME ERROR: 16 | PIN was wrong |
| Application to GSM | AT+CPIN=1234 | Enter PIN |
| GSM to application | OK | PIN was good |
| Application to GSM | AT+CLCK= ? | request supported facilities |
| GSM to application | +CLCK : ("PS","SC","AO","OI", "OX", "AI", "IR","AB", "AG","AC", "WNL") OK | supported facilities |
| Application to GSM | AT+CLCK="WNL",1,12345678 | Activate network lock |
| GSM to application | OK | Network lock activated |

## Change password		+CPWD

- This command shall be used by the application to change the PIN

The facility "SC" means SIM card. Other network facilities (Call barring, SIM or network lock) will use the same AT command.

For the network lock ("WNL"), the unlock is forbidden after 10 failed attempts to disable (unlock) the network lock with an incorrect password.

| Application to GSM | AT+CPWD=? | Possible values |
| --- | --- | --- |
| GSM to application | +CPWD: ("PS", 8),("SC", 8),("AO", 4),("OI", 4), ("OX", 4),("AI", 4),("IR", 4), ("AB", 4), ("AG", 4), ("AC", 4), ("WNL", 8) OK | PIN must be on 8 digits maximum |
| Application to GSM | AT+CPWD="SC",1234,5555 | change PIN |
| GSM to application | OK | PIN iwas good |
| Application to GSM | AT+CPWD="SC",1234,5555 | Change PIN |
| GSM to application | +CME ERROR: 16 | PIN was wrong |
| Application to GSM | AT+CPIN=5555 | Enter PIN |
| GSM to application | OK | PIN was good |
| Application to GSM | AT+CPWD="WNL",12345678, 00000000 | change NCK |
| GSM to application | OK | NCK changed for net lock |

## Select phonebook memory storage +CPBS

This command selects phonebook memory storage. The only available phonebook is the SIM phonebook (" SM ")

| Application to GSM | AT+CPBS=”SM” | Selected memory is in SIM |
| --- | --- | --- |
| GSM to application | OK | SIM phonebook is selected |
| Application to GSM | AT+CPBS=? | Possible values |
| GSM to application | +CPBS: ("SM") OK | SIM phonebook supported |
| Application to GSM | AT+CPBS ? | Status |
| GSM to application | +CPBS : "SM",10,20 OK | SIM phonebook selected, 10 used locations, 20 locations |

## Read phonebook entries +CPBR

This command returns phonebook entries for a location range from the current phonebook memory storage selected with +CPBS.

| Application to GSM | AT+CPBR=? | Test command |
| --- | --- | --- |
| GSM to application | +CPBR: (1-50), 20,10 OK | 50 locations (from 1 to 50), max length of 20 for phone 10 characters max for the associated text |
| Application to GSM | AT+CPBR=12,14 | Read entries from 12 to 14 |
| GSM to application | +CPBR : 12,"112",129,"Emergency" +CPBR : 13,"+331290909”, 145"Fred" +CPBR : 14,”0146290808”, 129,”Zazi" OK | Display locations 12,13,14 with Location, Number, Type, Text |
| Application to GSM | AT+CPBR=10 | Read entry 10 |
| GSM to application | +CPBR :10,"0146290921”,129,"Rob" OK | Display location 10 |
| Application to GSM | AT+CPBR=52 | Read entry 52 (wrong) |
| GSM to application | +CME ERROR: 21 | Invalid index |

## Find phonebook entries +CPBF

This command returns phonebook entries wich alphanumeric field start with a given string.

| Application to GSM | AT+CPBF=? | Test command |
| --- | --- | --- |
| GSM to application | +CPBF: 20,10 OK | max length of 20 for phone 10 characters for the text |
| Application to GSM | AT+CPBF="E" | Read entries with "E" |
| GSM to application | +CPBF: 12,"112",129,"Emergency" +CPBF: 15,"+331290101",145,"Eric" OK | Display locations with text field starting with "E" |
| Application to GSM | AT+CPBF="H" | Read entries with "H" |
| GSM to application | +CME ERROR: 22 | Entry not found |

## Write phonebook entiy +CPBW

This command writes phonebook entry in location number <index> in the current phonebook memory storage.

| Application to GSM | AT+CPBW=? | Test command |
| --- | --- | --- |
| GSM to application | +CPBW: (1-50),20,(129,145),10 OK | 50 locations, phone length=20, TON/NPI of 129 or 145, text length=10 |
| Application to GSM | AT+CPBW=3 | Erase location 3 |
| GSM to application | OK | Location 3 erased |
| Application to GSM | AT+CPBW=5,"112",129 ,"SOS" | Write at location 5 |
| GSM to application | OK | Location 5 written |
| Application to GSM | AT+CPBW=5,"01290917",129,"Jacky" | Overwrite location 5 |
| GSM to application | OK | Location 5 is overwritten |
| Application to GSM | AT+CPBW=,"+33145221100",145,"SOS" | Write at the first free location |
| GSM to application | OK | Free location is written |
| Application to GSM | AT+CPBW=,"0345221100",129,"SOS" | Write at the first free location |
| GSM to application | +CME ERROR: 20 | Phonebook full |
| Application to GSM | AT+CPBW=57,"112",129 ,"WM" | Write at loc 57 (wrong) |
| GSM to application | +CME ERROR: 21 | Invalid index |
| Application to GSM | AT+CPBW=7,"012345678901234567890",129,"FAL" | Write at loc 7 a long Phone number (21 digits) |
| GSM to application | +CME ERROR: 26 | Phone too long |
| Application to GSM | AT+CPBW=7,"0122334455",129,"LEIPOLDT" | Write at loc 7 a long Text (11 characters) |
| GSM to application | +CME ERROR: 24 | Text too long |

## Advice of charge +CAOC

This refers to Advice of Charge supplementary service (GSM 02.24 [] and GSM 02.86 []) that enables subscriber to get information about the cost of calls. With <mode>=0, the execute command returns the current call meter value (CCM) from the ME.

If AOC is supported, the command also includes the possibility to enable an unsolicited event reporting of the CCM information. The unsolicited result code +CCCM: <ccm> is sent when the CCM value changes. Deactivation of the unsolicited event reporting is made with the same command.

If AOC is supported, the Read command indicates whether the unsolicited reporting is activated or not.

Defined values

<mode>:

0	query CCM value

1	deactivate the unsolicited reporting of CCM value

2	activate the unsolicited reporting of CCM value

<ccm>: string type; three bytes of the current call meter value in hexadecimal format

(e.g. "00001E" indicates     decimal value 30); value is in home units and bytes are similarly coded as ACMmax value in the SIM

| Application to GSM | AT+CAOC=0 | Query CCM value |
| --- | --- | --- |
| GSM to application | +CAOC : "000A08" OK | Display Current Call Meter value (CCM = 2568) |
| Application to GSM | AT+CAOC=1 | Deactivate unsolicited report of CCM value |
| GSM to application | OK | CCM report deactivated |
| Application to GSM | AT+CAOC=2 | Activate unsolicited report of CCM value |
| GSM to application | OK | CCM report activated |
| Application to GSM | AT+CAOC ? | Request mode |
| GSM to application | +CAOC : <mode> OK | Display unsolicited report mode (1 or 2) |
| Application to GSM | AT+CAOC= ? | Request supported modes |
| GSM to application | +CAOC : (0-2) OK | 0,1,2 modes supported |

## Accumulated call meter +CACM

Set command resets the Advice of Charge related accumulated call meter value in SIM file EFACM. ACM contains the total number of home units for both the current and preceding calls. SIM PIN2 is required to reset the value. If setting fails in an ME error, +CME ERROR: <err> is returned.

Read command returns the current value of ACM.

Then ACM value (entered or displayed) is in hexadecimal format with 6 digits.

| Application to GSM | AT+CACM ? | Request ACM value |
| --- | --- | --- |
| GSM to application | +CACM : "000400" OK | Display ACM value (ACM = 1024) |
| Application to GSM | AT+CACM= 1234 | Request ACM reset, real PIN2 is "1234" |
| GSM to application | OK | ACM value is reset |
| Application to GSM | AT+CACM= 0000 | Request ACM reset with wrong PIN2 value |
| GSM to application | +CME ERROR : 17 | SIM PIN2 required |
| Application to GSM | AT+CACM ? | Request ACM value |
| GSM to application | +CACM : "000000" OK | Display ACM value (ACM = 0) |

## Accumulated call meter maximum +CAMM

Set command sets the Advice of Charge related accumulated call meter maximum value in SIM file EFACMmax. ACMmax contains the maximum number of home units allowed to be consumed by the subscriber. When ACM (refer +CACM) reaches ACMmax, then calls are prohibited. SIM PIN2 is required to set the value. If setting fails in an ME error, +CME ERROR: <err> is returned.

Read command returns the current value of ACMmax.

Them ACMmax value (entered or displayed) is in hexadecimal format with 6 digits.

| Application to GSM | AT+CAMM= "000400", 1234 | Request ACMmax update, real PIN2 is "1234" |
| --- | --- | --- |
| GSM to application | OK | ACMmax updated to 1024 |
| Application to GSM | AT+CAMM= "000400",0000 | Request ACMmax update, real PIN2 is "1234" |
| GSM to application | +CME ERROR : 17 | SIM PIN2 required |
| Application to GSM | AT+CAMM ? | Request ACMmax value |
| GSM to application | +CAMM : "000400" OK | ACMmax=1024 |

## Price per unit and currency table +CPUC

Description

Set command sets the parameters of Advice of Charge related price per unit and currency table in SIM file EFPUCT. PUCT information can be used to convert the home units (as used in +CAOC, +CACM and +CAMM) into currency units. SIM PIN2 is required to set the parameters. If setting fails in an ME error, +CME ERROR: <err> is returned.

| Application to GSM | AT+CPUC= "FFR", "0.82", 1234 | Request Currency and Price per unit update |
| --- | --- | --- |
| GSM to application | OK | ACMmax updated to 1024 |
| Application to GSM | AT+CPUC= "FFR", "0.82", 1111 | Request Currency and PPU update (wrong PIN2) |
| GSM to application | +CME ERROR : 17 | SIM PIN2 required |
| Application to GSM | AT+CPUC? | Request Currency & Price |
| GSM to application | +CPUC: "FFR", "0.82" OK | Currency= "FFR" Price per unit= "0.82" |

# AT commands for short messages

We recommend the GSM 07.05 from the ETSI in France, because there is a lot of information about the SMS handling in the GSM networks in.

## Parameters definition

<da> is the destination address.

<dcs> is the data coding scheme and is coded like in document [5].

<dt> is the discharge time.

<fo> is coded like first octet of SMS-SUBMIT in document [4].

<index> is the place of storage in memory.

<length> is the number of characters in text mode or the length of data binary string in PDU mode.

<mr> is the message reference.

<oa> is the originator address.

<pid> is the protocol identifier.

<pdu> is the binary string coded as specified in document [4]

<ra> is the recipient address.

<sca> is the address of the service center.

<scts> is the service center time stamp.

<st> is the status.

<stat> is the status of message in memory.

<tooa> is the integer format of <oa>.

<tora> is the integer format of <ra>.

<tosca> is the integer format of <sca>.

<vp> is the validity period of the message.

## Select message service		+CSMS

The supported services are GSM originated and terminated short message services. The Cell broadcast service is not supported yet.

| Application to GSM | AT+CSMS? | Current service |
| --- | --- | --- |
| GSM to application | +CSMS: 0, 1, 1, 0 OK | GSM 03.40 et 03.41, SMO and SMT supported, SMCB not supported |
| Application to GSM | AT+CSMS=? | Possible service |
| GSM to application | +CSMS: (0) OK | Only GSM 03.40 et 03.41 is possible |

## Preferred Message Storage	+CPMS

The incoming messages are automatically stored in the SIM.

| Application to GSM | AT+CPMS? | Read it |
| --- | --- | --- |
| GSM to application | +CPMS: "SM", 3, 10 OK | 3 messages are stored in SIM. 10 is the total available SIM memory. |
| Application to GSM | AT+CPMS=? | Possible storage |
| GSM to application | +CPMS: "SM" OK | Only SIM is possible |

## Preferred Message Format	+CMGF

The formats implemented are the text mode and the PDU mode.

In PDU mode, a complete SMS Message including all header information is passed as a binary string (document [4] for the format of the binary string).

In Text mode, every commands and responses are in ASCII characters.

The chosen format is stored in E2PROM by the command +CSAS.

| Application to GSM | AT+CMGF ? | current message format |
| --- | --- | --- |
| GSM to application | +CMGF: 1 OK | Text mode |
| Application to GSM | AT+CMGF=? | Possible message format |
| GSM to application | +CMGF: (0-1) OK | Text or PDU modes are available |

- Example to send a SMS Message in PDU mode

| Application to GSM | AT+CMGF=0 | PDU message format |
| --- | --- | --- |
| GSM to application | OK | PDU mode valid |
| Application to GSM | AT+CMGS=14<CR>01F60691214365000004C9E9340B | Send MSG |
| GSM to application | +CMGS: 246 OK | MSG correctly sent |

14 is the length of octets of the PDU buffer coded as GSM 03.40 . In this case this buffer is 0x01 0xF6 0x06 0x91 0x21 0x43 0x65 0x00 0x00 0x04 0xC9 0xE9 0x34 0x0B which means regarding to GSM 03.40 :

TP-MIT:	0x01 (SMS-SUBMIT)

<mr>:		0xF6

<da>:		0x06 0x91 0x21 0x43 0x65 (+123456)

<pid>:		0x00

<dcs>:		0x00 (Data coding scheme : 7 bits alphabet chosen)

TP-UDL:	0x04 (User data length)

TP-UD:	0xC9 0xE9 0x34 0x0B (User data : ISSY)

TPDU in hexadecimal format must be converted into two ascii characters. E.G octet with hexadecimal value 0x2A is presented to the mobile as two characters ‘2’ (ascii 50) and ‘A’ (ascii 65).

## Save settings	+CSAS

All settings specified in command +CSCA, +CMGF, +CSMP, +CNMI, +CSDH are stored in E2PROM if the SIM card is phase 1 or in the SIM card if this one is phase 2.

| Application to GSM | AT+CSAS | Store |
| --- | --- | --- |
| GSM to application | OK | command correct |

## Restore settings	+CRES

All settings specified in command +CSCA, +CMGF, +CSMP, +CNMI, +CSDH are restored from E2PROM if the SIM card is phase 1 or from the SIM card if this one is phase 2.

| Application to GSM | AT+CRES | Restore |
| --- | --- | --- |
| GSM to application | OK | command correct |

## Show text Mode Parameters	+CSDH

This command gives more information in text mode result codes. These information’s are in brackets in commands +CMTI,+CMT,+CDS,+CMGR, +CMGL.

| Application to GSM | AT+CSDH? | Current value |
| --- | --- | --- |
| GSM to application | +CSDH: 0 OK | do not show header values |

## Select TE character set	+CSCS

This commands informs the ME which character set is used by the TE. ME is then able to convert character strings correctly. This is used for commands +CMGS in text mode.

| Application to GSM | AT+CSCS="GSM" | GSM default alphabet |
| --- | --- | --- |
| GSM to application | OK | Command valid |
| Application to GSM | AT+CSCS="PCCP437" | PC character set Code Page 437 |
| GSM to application | OK | Command valid |

## New message indication		+CNMI

This command selects the procedure how receiving the message from the network. The application must send the following command :

- AT+CNMI=<mode>,<mt>,<bm>,<ds>,<bfr>

<mode> (controls the processing of unsolicited result codes):

| 0 | Buffer unsolicited result codes in the TA. If TA result code buffer is full, indications can be buffered in some other place or the oldest indications may be discarded and replaced with the new received indications. |
| --- | --- |
| 1 | Discard indication and reject new received message unsolicited result codes when TA-TE link is reserved. Otherwise forward them directly to the TE. |
| 2 | Buffer unsolicited result codes in the TA when TA-TE link is reserved and flush them to the TE after reservation. Otherwise forward them directly fo the TE. |
| 3 | Forward unsolicited result codes directly to the TE. TA-TE link specific inband used to embed result codes an data when TA is in on-line data mode. |

<mt> (sets the result code indication routing for SMS-DELIVERs):

| 0 | No SMS-DELIVER indications are routed- |
| --- | --- |
| 1 | SMS-DELIVERs are routed using unsolicited code: +CMTI: ‹‹SM›› <index> |
| 2 | SMS-DELIVERs (except class 2 messages) are routed using unsollicited code: +CMT: <pdu> (if PDU mode chosen) or +CMT <oa>, <scts> [<tooa>, <fo>, <pid>, <dcs>, <sca>, <tosca>, <length>]<CR><LF><data> |
| 3 | Class 3 SMS-DELIVERs are routed directly using code in <mt>=2; Message of other classes result in indication <mt>=1 |

<bm>: set the rules for storing received CBMs Types depend on ist coding scheme, the setting of Select CBM Types (+CSCB commands) and <bm>.

| 0 | No CBM indications are routed to the TE. |
| --- | --- |
| 1 | New CBMs are routed directly to the TE using unsolicited result code +CBM: <length><CR><LF><pdu> (PDU mode) or +CBM:<sn>, <mid>, <dcs>, <page>, <pages><CR><LF><data> (Ttext mode) |

<ds> (for SMS-STATUS-REPORTs):

| 0 | No SMS-STATUS-REPORTs are routed. |
| --- | --- |
| 1 | SMS-STATUS_REPORTs are routed using unsolicited code: +CDS: <pdu> (if PDU mode chosen) or +CDS: <fo>, <mr>, [<ra>, <tora>], <scts>, <dt>, <st> |

<bfr>

| 0 | TA buffer of unsolicited result codes defined within this command is flushed to the TE when <mode> 1...3 is entered (OK response shall be given before flushing the codes) |
| --- | --- |
| 1 | TA buffer of unsolicited result codes defined within this command is cleared when <mode> 1...3 is entered. |

Examples:

| Application to GSM | AT+CNMI=0,1,0,0,0 | <mt>=1 |
| --- | --- | --- |
| GSM to application | OK |  |
| GSM to application | +CMTI : « SM », 1 | Message received |
| Application to GSM | AT+CNMI=0,2,0,0,0 | <mt>=2 |
| GSM to application | OK |  |
| GSM to application | +CMT :"123456","13/01/96 12h30m00s",129,4,32,240,"15379",129,5<CR><LF> HELLO | Message received |
| Application to GSM | AT+CNMI=0,0,0,1,0 | <ds>=1 |
| GSM to application | OK |  |

| Application to GSM | AT+CMGS="+33146290800"<CR> Hello, how are you ?^Z | Send a message in text mode |
| --- | --- | --- |
| GSM to application | +CMGS: 116 OK | Successful transmission |
| GSM to application | +CDS:2,116+33146290800", 145,"13/01/96 12h30m00s","14/01/96 20h51m05s",16 |  |

## Read message		+CMGR

This command allows the application to read incoming stored messages.

- AT+CMGR=<index>

+CMGR=<stat>,<oa>,<scts>[,<tooa>,<fo>,<pid>,<dcs>,<sca>,<tosca>, <length>]<CR><LF><data>		(for SMS-DELIVER only)

This command is also available in PDU mode.

+CMGR=<stat>,<length<CR><LF><pdu>

Example :

| GSM to application | +CMTI: "SM",1 | New message received |
| --- | --- | --- |
| Application to GSM | AT+CMGR=1 | read the message |
| GSM to application | +CMGR: "REC UNREAD", "43322449"<CR> To be or not to be! OK |  |
| Application to GSM | AT+CMGR=1 | read again the message |
| GSM to application | +CMGR: "REC READ", "43322449",20<CR> To be or not to be! OK |  |
| Application to GSM | AT+CMGR=2 | read + bad index |
| GSM to application | +CMS ERROR: 321 | error : invalid index |
| Application to GSM | AT+CMGR=1 | in PDU mode |
| GSM to application | +CMGR: "REC READ", <length><CR><LF><pdu> OK |  |

## List message	+CMGL

This command allows the application to read stored messages, by indicating the type of the message to read.

- AT+CMGL=<stat>

<stat> (status of message in memory) :

| 0 | "REC UNREAD" ( received unread message ) |
| --- | --- |
| 1 | "REC READ"( received read message ) |
| 2 | "STO UNSENT"( stored unsent message ) |
| 3 | "STO SENT"( stored sent message ) |
| 4 | "ALL"( all messages ) |

+CMGL=<index>,<stat>,<da/oa>[,<alpha>,<scts>,<tooa/toda>,<length>]<CR><LF><data>

This command is also available in PDU mode.

+CMGL=<index>,<stat>,<length<CR><LF><pdu>

Examples :

| Application to GSM | AT+CMGL=0 | read the message |
| --- | --- | --- |
| GSM to application | +CMGL: 1, "REC UNREAD", "43322449"<CR> To be or not to be! +CMGL: 3, "REC UNREAD", "46290800"<CR> Be happy! OK |  |
| Application to GSM | AT+CMGL=1 | read again the message |
| GSM to application | +CMGL: 2, "REC READ", "43322449",20<CR> Keep cool OK |  |
| Application to GSM | AT+CMGL=2 | read + bad index |
| GSM to application | +CMS ERROR: 321 | error : invalid index |

## Send message		+CMGS

The <address> field is the address of the terminal network to whom the message is sent ; <exit method> is ^Z : ASCII 26. The text can contain all existing character except ^Z.

Example of use :

| Application to GSM | AT+CMGS="+33146290800"<CR> Hello, how are you ?^Z | Send a message in text mode |
| --- | --- | --- |
| GSM to application | +CMGS : <mr> OK | Successful transmission |
| Application to GSM | AT+CMGS=<length><CR> <pdu>^Z | Send a message in PDU mode |
| GSM to application | +CMGS : <mr> OK | Successful transmission |

The message reference <mr> which is returned back to the application is allocated by the GSM module. This number begins with 0 and is incremented by one for each outgoing message (successful and failure case) ; it is cyclic on one byte (0 follows 255).

Note : this number is not a storage number - outgoing messages are not stored.

## Set Text Mode Parameters		+CSMP

This command shall be used to select value for the <vp>, <pid>, the <dcs>. The application must send the following command :

- AT+CSMP=<fo>, <vp>, <pid>,<dcs>

In text mode <vp> is coded only in "relative" format. This means one octet (see document [4] for more details).

| Application to GSM | AT+CSMP? | current values |
| --- | --- | --- |
| GSM to application | +CSMP: 0,0,0,0 OK | no validity period dcs = PCCP437 alphabet ( 8 bits -> 7 bits ) |
| Application to GSM | AT+CSMP=16,32,64,244 | validity period = 32 dcs = GSM 8 bits alphabet |
| GSM to application | OK | command correct |

## Delete message		+CMGD

This command shall be used after a read-command in order to delete the received stored message.

For example :

| GSM to application | +CMTI: "SM",3 | New message received |
| --- | --- | --- |
| Application to GSM | AT+CMGR=3 | Read it |
| GSM to application | +CMGR: "REC UNREAD", "43322449",20<CR> To be or not to be! OK |  |
| Application to GSM | AT+CMGD=3 | Delete it |
| GSM to application | OK |  |

## Service center address	+CSCA

This command shall be used to indicate to which service center the message has to be sent.

The GSM module has no default value for this address. If the application tries to send a message without having indicated the service center address, an error will be generated.

So, the application has to indicate this address at initialization. This address is then valid all the time. The application may change it if needed.

For example, let us suppose that no AT+CSCA has been already sent.

| Application to GSM | AT+CMGS="+33146290800"<CR> Hello, how are you ?^Z | Send a message |
| --- | --- | --- |
| GSM to Application | +CMS ERROR : 330 | Service center unknown |
| Application to GSM | AT+CSCA="96741234" | Service center initialization |
| GSM to application | OK |  |
| Application to GSM | AT+CMGS="+33146290800"<CR> Hello, how are you ?^Z | Send again the same message |
| GSM to application : | +CMGS : 1 OK | Successful transmission |

# AT commands for Supplementary Services

## Call forwarding		+CCFC

This commands allows control of the call forwarding supplementary service. The command to manage this functionality is :

AT+CCFC=<reason>,<mode>[, <number>]

* <reason> :

| 0 | Unconditional |
| --- | --- |
| 1 | mobile busy |
| 2 | no reply |
| 3 | not reachable |
| 4 | all call forwarding |
| 5 | all conditional call forwarding |

* <mode> :

| 2 | Interrogate |
| --- | --- |
| 3 | Registration |
| 4 | Erasure |

| Application to GSM | AT+CCFC=0,3,146290800 | register call forwarding unconditional |
| --- | --- | --- |
| GSM to application | OK | command valid |
| Application to GSM | AT+CCFC=0,2 | interrogate call forwarding unconditional |
| GSM to application | +CCFC=1,7,146290800 OK | call forwarding active |
| Application to GSM | AT+CCFC=0,4 | erase call forwarding unconditional |
| GSM to application | OK | command valid |

## Call barring	+CLCK

This commands allows control of the call barring supplementary service. The command to manage this functionality is :

- AT+CLCK=<fac>,<mode>, <password>

<fac> :see 4.10 chapter

“AO”,”OI”,”OX”	for outgoing calls barring

“AI”,”IR”		for incoming calls barring

“AG”,”AC”,”AB”	for all calls barring (<mode>=0 only)

<mode>

| 0 | Unlocks the facility |
| --- | --- |
| 1I | Locks the facility |

Password code must be on 4 digits maximum.

| Application to GSM | AT+CLCK="AO",1,1234 |  |
| --- | --- | --- |
| GSM to application | OK | command valid |
| Application to GSM | AT+CLCK="AO",0,5555 |  |
| GSM to application | +CME ERROR: 16 | wrong password |
| Application to GSM | AT+CLCK="AO",0,1234 |  |
| GSM to application | OK | command valid |

## Modify SS password		+CPWD

This command shall be used by the application to change the supplementary service password. The command to manage this functionality is :

- AT+CPWD=<fac>,<oldpassword>, <newpassword>

for <fac> see +CLCK command.

| Application to GSM | AT+CPWD="AO",1234,5555 | change password |
| --- | --- | --- |
| GSM to application | OK | Password was good |
| Application to GSM | AT+CPWD="AO",1234,5555 | Change password |
| GSM to application | +CME ERROR: 16 | password wrong |
| Application to GSM | AT+CPWD="AO",5555,1234 | change password |
| GSM to application | OK | Password was good |

## Call waiting	+CCWA

This command allows control of the call waiting supplementary service. The module supports only one communication. If a call waiting call happens, the module will disconnect it.

The command to manage this functionality is :

AT+CCWA=<n>,<mode>[, <class>]

* <n> : (result code presentation status in the TA)

| 0 | Disable |
| --- | --- |
| 1 | Enable |

* <mode> :

| 0 | Disable |
| --- | --- |
| 1 | Registratio |
| 2 | Interrogate |

* <class> :

| 1 | Voice |
| --- | --- |
| 2 | Data |
| 4 | fax |

| Application to GSM | AT+CCWA=1,1 | enable call waiting |
| --- | --- | --- |
| GSM to application | OK | command valid |
| Application to GSM | AT+CCWA=1,2 | interrogate call forwarding unconditional |
| GSM to application | +CCWA="x",7 OK | "x" = 1 active "x" = 0 not active |
| GSM to application | RING +CCWA: +146290800,145,7 NO CARRIER | incoming call number of the incoming call incoming call is disconnected |
| Application to GSM | AT+CCWA=1,0 | erase call waiting |
| GSM to application | OK | command valid |

## Calling line identification restriction	+CLIR

This command allows control of the calling line identification restriction supplementary service.

- AT+CLIR=<n>

* <n> : (temporary mode status in the TA)

| 0 | set presentation indicator according to the subscription of the CLIR service |
| --- | --- |
| 1 | CLIR invocation |
| 2 | CLIR suppression |

| Application to GSM | AT+CLIR=2 |  |
| --- | --- | --- |
| GSM to application | OK | command valid |
| Application to GSM | AT+CLIR? | Ask for current functionality |
| GSM to application | +CLIR: <n>,"y" OK | "y" as defined in the GSM TS 07 07 |

## Calling line identification presentation	+CLIP

This command allows control of the calling line identification presentation supplementary service :		AT+CLIP=<n>

* <n> : (result code presentation status in the TA)

| 0 | disable |
| --- | --- |
| 1 | enable |

| Application to GSM | AT+CLIP=1 | CLIP on |
| --- | --- | --- |
| GSM to application | OK | command valid |
| Application to GSM | AT+CLIP? | ask for current functionality |
| GSM to application | +CLIP: 1,"x" OK | "x" = 0 CLIP not provisioned "x" = 1 CLIP provisioned "x" = 2 unknown (no network ...) |
| GSM to application | RING +CLIP: 146290800,145 | incoming call |
| Application to GSM | AT+CLIP=0 | CLIP OFF |
| GSM to application | OK | command valid |

## Connected line identification presentation		+COLP

This command allows control of the connected line identification presentation supplementary service.

- AT+COLP=<n>

* <n> : (result code presentation status in the TA)

| 0 | disable |
| --- | --- |
| 1 | enable |

| Application to GSM | AT+COLP=1 | COLP on |
| --- | --- | --- |
| GSM to application | OK | command valid |
| Application to GSM | AT+COLP? | ask for current functionality |
| GSM to application | +COLP: 1,"x" OK | "x" = 0 COLP not provisioned "x" = 1 COLP provisioned "x" = 2 unknown (no network ...) |
| Application to GSM | ATD 01719045671; | outgoing call |
| GSM to application | +COLP: 146290928,145 OK |  |
| Application to GSM | AT+COLP=0 | COLP OFF |
| GSM to application | OK | command valid |

# AT commands for data

## Bearer type selection	+CBST

- The syntax of the command is  AT+CBST=<speed>,0,<ce>

The selection of the bearer impacts both incoming and outgoing calls. No data compression is provided and only asynchronous modem is supported.

| <speed> | Description | Modem type |
| --- | --- | --- |
| 0 | Autobauding | None |
| 1 | 300 bps | V.21 |
| 2 | 1200bps | V.22 |
| 3 | 1200/75 bps | V.23 |
| 4 | 2400 bpsV32 | V.22bis |
| 5 | 2400 bps | V.26ter |
| 6 | 4800 bps | V.32 |
| 7 | 9600 bps | V.32 |
| 8 | Specific |  |
| 65(*) | 300 bps | V.110 |
| 66(*) | 1200 bps | V.110 |
| 68 | 2400 bps | V.110 |
| 70 | 4800 bps | V.110 |
| 71 | 9600 bps | V.110 |

*() Only for non transparent mode

| <ce> | connection element |
| --- | --- |
| 0 | Transparent only |
| 1 | Non transparent only |
| 100 | Transparent preferred |
| 101 | Non transparent preferred |

| Application to GSM | AT+CBST=7,0,1 | ask for a bearer |
| --- | --- | --- |
| GSM to application | OK | bearer supported |
| Application to GSM | AT+CBST=81,0,0 | ask for a bearer |
| GSM to application | +CME ERROR : 4 | bearer not supported |

## Report control		+CR, +CRC

### Outgoing call	+CR

This command enables a more detailed service reporting, in case of data outgoing call. Before sending the CONNECT response to the application, the GSM module will precise the type of data connection that have been established.

These report types are :

| +CR: ASYNC | For asynchronous transparent |
| --- | --- |
| +CR: REL ASYNC | For asynchronous non-transparent |

| Application to GSM | AT+CR=0 | extended reports disabled |
| --- | --- | --- |
| GSM to application | OK | command valid |
| Application to GSM | AT+CR=1 | extended reports enabled |
| GSM to application | OK | command valid |

Default value is stored in E2PROM.

### Incoming call	+CRC

This command enables a more detailed ring indication, in case of incoming call (voice or data). Instead of the string "RING", an extended string is used to indicate which type of call is ringing (e.g.+CRING:VOICE).

These extended indications are :

| +CRING: ASYNC | for asynchronous transparent |
| --- | --- |
| +CRING: REL ASYNC | for asynchronous non-transparent |
| +CRING: VOICE | for normal speech. |
| +CRING : FAX | for fax calls |

| Application to GSM | AT+CRC=0 | extended reports disabled |
| --- | --- | --- |
| GSM to application | OK | command valid |
| Application to GSM | AT+CRC=1 | extended reports enabled |
| GSM to application | OK | command valid |

Default value is stored in E2PROM.

## Echo			E

Determines whether the mobile echoes (sends) the commands you enter to your computer screen as you type them..

| Application to GSM | ATE0 | no local echo |
| --- | --- | --- |
| GSM to application | OK | command valid |
| Application to GSM | ATE1 | local echo |
| GSM to application | OK | command valid |

## Fixed DTE rate			+IPR

This commands specifies the data rate at which the DCE will accept commands.

| Application to GSM | AT+IPR? |  |
| --- | --- | --- |
| GSM to application | +IPR: 9600 OK | current rate is 9600 bps |
| Application to GSM | AT+IPR=? |  |
| GSM to application | +IPR:(),(2400,9600,19200,38400,57600,115200) OK | possible values. No autodectable rate supported |
| Application to GSM | AT+IPR=38400 |  |
| GSM to application | OK | new rate is 38400 bps |

## Back to online mode		O

If you have established a connection and the mobile is in online command mode, this command allows to return to online data mode.

| Application to GSM | ATO |  |
| --- | --- | --- |
| GSM to application | OK | command valid |

## DCE response format	V

Determines the DCE response format, with or without header characters  <CR><LF>, and with the use of numeric result codes.

|  | V0 | V1 |
| --- | --- | --- |
| Information responses | <text><CR><LF> | <CR><LF> <text><CR><LF> |
| Result codes | <numeric code><CR> | <CR><LF> <verbose code><CR><LF> |

| Application to GSM | ATV0 | DCE transmits limited headers and trailers and numeric result codes |
| --- | --- | --- |
| GSM to application | 0 | command is valid (0 means OK) |
| Application to GSM | ATV1 | DCE transmits full headers and trailers and verbose response text |
| GSM to application | OK | command is valid |

## DTE-DCE character framing	+ICF

This command is used to determine the local serial port start-stop (asynchronous) character framing that the DCE shall use.

- AT+ICF=<format>,<parity>

* <format> :

| 0 | Autodetect | not supported |
| --- | --- | --- |
| 1 | 8 Data 2 Stop | not supported |
| 2 | 8 Data 1 Parity 1 Stop | not supported |
| 3 | 8 Data 1 Stop | supported |
| 4 | 7 Data 2 Stop | not supported |
| 5 | 7 Data 1 Parity 1 Stop | not supported |
| 6 | 7 Data 1 Stop | not supported |

* <parity> :

| 0 | Odd | not supported |
| --- | --- | --- |
| 1 | Even | not supported |
| 2 | Mark | not supported |
| 3 | Space | not supported |
| 4 | None | supported |

| Application to GSM | AT+ICF? |  |
| --- | --- | --- |
| GSM to application | +ICF: 3,4 OK | current values |
| Application to GSM | AT+ICF=? |  |
| GSM to application | +ICF: (1-6),(0-4) OK | possible values. |
| Application to GSM | AT+ICF=0,0 |  |
| GSM to application | OK | new values |

## Repeat last command		A/

| Application to GSM | A/ | repeat last command |
| --- | --- | --- |

## Default configuration		Z

Restores the configuration profile.

| Application to GSM | ATZ |  |
| --- | --- | --- |
| GSM to application | OK | command valid |

## DTE-DCE local flow control	+IFC

This command is used to control the operation of local flow control between the DTE and DCE.

AT+IFC=<DCE_by_DTE>,<DTE_by_DCE>

* < DCE_by_DTE > :

| 0 | None | not supported |
| --- | --- | --- |
| 1 | Xon/Xoff local circuit 103 | not supported |
| 2 | RTS | supported |
| 3 | Xon/Xoff global on circuit 103 | not supported |

* < DTE_by_DCE > :

| 0 | None | not supported |
| --- | --- | --- |
| 1 | Xon/Xoff circuit 104 | not supported |
| 2 | CTS | supported |

| Application to GSM | AT+IFC? |  |
| --- | --- | --- |
| GSM to application | +IFC: 2,2 OK | current values |
| Application to GSM | AT+IFC=? |  |
| GSM to application | +IFC: (2),(2) OK | possible values. |
| Application to GSM | AT+IFC=0,0 |  |
| GSM to application | OK | new values |

## Set DCD signal		&C

This commands controls the Data Carrier Detect (DCD) signal.

| Application to GSM | AT&C0 | DCD always on |
| --- | --- | --- |
| GSM to application | OK | command valid |
| Application to GSM | AT&C1 | DCD matches state of the remote modem's data carrier |
| GSM to application | OK | command valid |

## Set DTR signal		&D

This commands controls the Data Terminal Ready (DTR) signal.

| Application to GSM | AT&D0 | The DTR signal is ignored |
| --- | --- | --- |
| GSM to application | OK | command valid |
| Application to GSM | AT&D1 | Modem switches from data to command mode when DTR switches from on to OFF |
| GSM to application | OK | command valid |
| Application to GSM | AT&D2 | Upon DTR switch from on to OFF, the call is clear down |
| GSM to application | OK | command valid |

## Set DSR signal		&S

This commands controls the Data Set Ready (DSR) signal.

| Application to GSM | AT&S0 | DSR always on |
| --- | --- | --- |
| GSM to application | OK | Command valid |
| Application to GSM | AT&S1 | DSR off in command mode, DSR on in data mode |
| GSM to application | OK | Command valid |

## Save configuration		&W

This commands writes the active configuration to nonvolatile RAM.

| Application to GSM | AT&W | Writes current configuration to E2prom |
| --- | --- | --- |
| GSM to application | OK | Command valid |

## Radio link protocol parameters	+CRLP

This command allows to change the radio link protocol parameters used for non transparent data transmission.

AT+CRLP=<up-window size>,<down-window size>,<acknowledgement timer>,<retransmission attempts>,<reset allowed>

| Application to GSM | AT+CRLP=10,10,90,10,0 | set new parameters |
| --- | --- | --- |
| GSM to application | OK | command valid |
| Application to GSM | AT+CRLP? | current values |
| GSM to application | AT+CRLP: 10,10,90,10 OK | command valid |

# Other AT commands

## V.25 ter recommendation

All the remaining basic commands are implemented in FUNKANLAGEN LEIPOLDT OHG's AT parser. An  "OK"  string will be sent back but no further software treatment will happen.

All commands about modulation control, error control and data compression are not recognized. An "ERROR" string will be sent back.

## GSM 07.05 recommendation

All the remaining commands are not implemented, so an “ERROR” string will be sent back

## GSM 07.07 recommendation

All the remaining commands are not implemented, so an “ERROR” string will be sent back.

# Failure result codes

## Report Mobile Equipement errors	+CMEE

This command disables or enables the use of result code +CME ERROR : <xxx> or +CMS ERROR :<xxx>

| Application to GSM | AT+CMEE=0 | disable « +CME ERROR : <xxx> » and use « ERROR » instead |
| --- | --- | --- |
| GSM to application | OK |  |
| Application to GSM | AT+CMEE=1 | enable « +CME ERROR : <xxx> » |
| GSM to application | OK |  |

## Extended error report	+CEER

If the last call setup (originating or answering) fails, this command gives the reason of the call release.

| Application to GSM | ATD123456789 ; | outgoing voice call |
| --- | --- | --- |
| GSM to application | NO CARRIER |  |
| Application to GSM | AT+CEER | ask for reason of release |
| GSM to application | +CEER : Error <xxx> OK | <xxx> is the cause information element values from GSM recommendation 04.08 |

The cause information element from GSM 04.08 is given below.

## Mobile equipment error result code : +CME ERROR: xxx

<error> is defined as below :

| 0 | Phone failure |
| --- | --- |
| 3 | Operation not allowed |
| 4 | Operation not supported |
| 10 | SIM not inserted |
| 11 | SIM PIN required |
| 12 | SIM PUK required |
| 13 | SIM failure |
| 16 | Incorrect password |
| 22 | Not found |
| 24 | Text string too long |
| 26 | Dial string too long |
| 30 | No network service |
| 256 | protocol stack bad state |
| 257 | Bad cell ( not in the synchronized ones |
| 258 | Lost cell ( due to DSF...) |

## Message service failure result code:  +CMS ERROR :<er>

<er> is defined as below :

| 1 to 127 | error cause values from the GSM recommendation 04.11 Annex E-2 |
| --- | --- |
| 301 | SMS service of ME reserved |
| 302 | operation not allowed |
| 303 | operation not supported |
| 305 | invalid text mode parameter |
| 313 | SIM failure |
| 321 | Invalid memory index |
| 322 | SIM memory full |
| 330 | SC address unknown |

Error cause values from GSM 04.11 are given below.

## Cause information element values from GSM recommendation 04.08

| Cause value | Diagnostic |
| --- | --- |
| 1 | Unassigned (unallocated) number |
| 3 | No route to destination |
| 6 | Channel unacceptable |
| 8 | Operator determined barring |
| 16 | Normal call clearing |
| 17 | User busy |
| 18 | No user responding |
| 19 | User alerting, no answer |
| 21 | Call rejected |
| 22 | Number changed |
| 26 | Non selected user clearing |
| 27 | Destination out of order |
| 28 | Invalid number format (incomplete number) |
| 29 | Facility rejected |
| 30 | Response to STATUS ENQUIRY |
| 31 | Normal, unspecified |
| 34 | No circuit/channel available |
| 38 | Network out of order |
| 41 | Temporary failure |
| 42 | Switching equipment congestion |
| 43 | Access information discarded |
| 44 | requested circuit/channel not available |
| 47 | Resources unavailable, unspecified |
| 49 | Quality of service unavailable |
| 50 | Requested facility not subscribed |
| 55 | Incoming calls barred with in the CUG |
| 57 | Bearer capability not authorized |
| 58 | Bearer capability not presently available |
| 63 | Service or option not available, unspecified |
| 65 | Bearer service not implemented |
| 68 | ACM equal to or greater than ACMmax |
| 69 | Requested facility not implemented |
| 70 | Only restricted digital information bearer capability is available |
| 79 | Service or option not implemented, unspecified |
| 81 | Invalid transaction identifier value |
| 87 | User not member of CUG |
| 88 | Incompatible destination |
| 91 | Invalid transit network selection |
| 95 | Semantically incorrect message |
| 96 | Invalid mandatory information |
| 97 | Message type non-existent or not implemented |
| 98 | Message type not compatible with protocol state |
| 99 | Information element non-existent or not implemented |
| 100 | Conditional IE error |
| 101 | Message not compatible with protocol state |
| 102 | Recovery on timer expire |
| 111 | Protocol error, unspecified |
| 127 | Interworking, unspecified |
| 252 | Call barring on outgoing calls |
| 253 | Call barring on incoming calls |
| 254 | Call impossible |
| 255 | Lower layer failure |

All other values in the range 0 to 31 shall be treated as cause 31.

All other values in the range 32 to 47 shall be treated as cause 47.

All other values in the range 48 to 63 shall be treated as cause 63.

All other values in the range 64 to 79 shall be treated as cause 79.

All other values in the range 80 to 95 shall be treated as cause 95.

All other values in the range 96 to 111 shall be treated as cause 111.

All other values in the range 112 to 127 shall be treated as cause 127.

## GSM 04.11 Annex E-2:  Mobile originating SM-transfer

- Cause no 1:  "Unassigned (unallocated) number"

This cause indicates that the destination requested by the Mobile Station cannot be reached because, although the number is in a valid format, it is not currently assigned (allocated).

- Cause no 8: "Operator determined barring"

This cause indicates that the MS has tried to send a mobile originating short message when the MS's network operator or service provider has forbidden such transactions.

- Cause no 10:  "Call barred"

This cause indicates that the outgoing call barred service applies to the short message service for the called destination.

- Cause no 21:  "Short message transfer rejected"

This cause indicates that the equipment sending this cause does not wish to accept this short message, although it could have accepted the short message since the equipment sending this cause is neither busy nor incompatible.

- Cause no 27:  "Destination out of service"

This cause indicates that the destination indicated by the Mobile Station cannot be reached because the interface to the destination is not functioning correctly.  The term "not functioning correctly" indicates that a signaling message was unable to be delivered to the remote user; e.g., a physical layer or data link layer failure at the remote user, user equipment off-line, etc.

- Cause no 28:  "Unidentified subscriber"

This cause indicates that the subscriber is not registered in the PLMN (i.e. IMSI not known)

- Cause no 29:  "Facility rejected"

This cause indicates that the facility requested by the Mobile Station is not supported by the PLMN.

- Cause no 30:  "Unknown subscriber"

This cause indicates that the subscriber is not registered in the HLR (i.e. IMSI or directory number is not allocated to a subscriber).

- Cause no 38:  "Network out of order"

This cause indicates that the network is not functioning correctly and that the condition is likely to last a relatively long period of time; e.g., immediately reattempting the short message transfer is not likely to be successful.

- Cause no 41:  "Temporary failure"

This cause indicates that the network is not functioning correctly and that the condition is not likely to last a long period of time; e.g., the Mobile Station may wish to try another short message transfer attempt almost immediately.

- Cause no 42:  "Congestion"

This cause indicates that the short message service cannot be serviced because of high traffic.

- Cause no 47:  "Resources unavailable, unspecified"

This cause is used to report a resource unavailable event only when no other cause applies.

- Cause no 69:  "Requested facility not implemented"

This cause indicates that the network is unable to provide the requested short message service.

- Cause no 81:  "Invalid short message transfer reference value"

This cause indicates that the equipment sending this cause has received a message with a short message reference which is not currently in use on the MS-network interface.

- Cause no 95:  "Invalid message, unspecified"

This cause is used to report an invalid message event only when no other cause in the invalid message class applies.

- Cause no 96:  "Invalid mandatory information"

This cause indicates that the equipment sending this cause has received a message where a mandatory information element is missing and/or has a content error (the two cases are indistinguishable).

- Cause no 97:  "Message type non-existent or not implemented"

This cause indicates that the equipment sending this cause has received a message with a message type it does not recognize either because this is a message not defined or defined but not implemented by the equipment sending this cause.

- Cause no 98:  "Message not compatible with short message protocol state"

This cause indicates that the equipment sending this cause has received a message such that the procedures do not indicate that this is a permissible message to receive while in the short message transfer state.

- Cause no 99:  "Information element non-existent or not implemented"

This cause indicates that the equipment sending this cause has received a message which includes information elements not recognized because the information element identifier is not defined or it is defined but not implemented by the equipment sending the cause.

However, the information element is not required to be present in the message in order for the equipment sending the cause to process the message.

- Cause no 111: "Protocol error, unspecified"

This cause is used to report a protocol error event only when no other cause applies.

- Cause no 127: "Interworking, unspecified"

This cause indicates that there has been interworking with a network which does not provide causes for actions it takes; thus, the precise cause for a message which is being send cannot be ascertained.

# Fax management

The firmware you have been provided with includes the fax service (Group3, Class1)

The fax functionalities of this version have been tested and they are fully functional in several countries with the following fax softwares: Delrina Winfax, Trio, MS Winfax, However, the fax functions might work in other coutries and with other fax softwares.

## Setting up the module for fax:

The GSM unit as well as the PC fax application have to be properly set up prior to send/receive faxes-

The only thing to do is to change the serial link rate as follows:

- Launch any terminal application on PC and set it up to 9600 baud.

- It is recommended to set all extended messages of the GSM to 0:

- AT+CLIR=0

- AT+CLIP=0

- AT+COLP=0

- AT+CR=0

- AT+CRC1=0

- AT+CMEE=0

- From the terminal application type in: AT+IPR=2400; &W<enter>

The GSM unit should reply OK (if not, type in `AT<enter>`as long as it does not answer ´OK` and start again stage b).

## Setting up the PC fax application:

### Delrina WinFax v4.0

The recommended fax application is Delrina WinFax v4.0.

It should configured as follows (menu Setup/Fax Modem Setup):

- Port com 1

- Model: Generic Class 1 (HARDWARE FLOW CONTROL). Above all, do not select ‹‹Generic Class 1›› (Software flow control).

- Init: clear default string

- Reset: clear default string

- Maximum transmit Rate: 9600 baud.

- Others settings are of no interest for the GSM unit: they can be modified.

## Restoring the module default profile for data call

If the module is configured as stated in 1/ then the serial rate is 2400 baud which of course limits the overall thoughput in support of 4800/9600 data calls.

Therefore, the serial rate should be set back to 9600 (or better 19200) baud by issueing: AT+IPR=9600; &W <enter>

# General

- IMPORTANT

FOR THE EFFICIENT AND SAFE OPERATION OF

YOUR GSM MODEM

READ THIS INFORMATION BEFORE USE

Your GSM modem is one of the most exciting and innovative electronic products ever developed. With it you can stay in contact with your office, your home, emergency services, and others, wherever service is provided.

- GENERAL

Your modem utilizes the GSM standard for cellular technology. GSM is a newer radio frequency (« RF ») technology than the current FM technology that has been used for radio communications for decades. The GSM standard has been established for use in the European community and elsewhere.

Your modem is actually a low power radio transmitter and receiver. It sends out and receives radio frequency energy. When you use your modem, the cellular system handling your calls controls both the radio frequency and the power level of your cellular modem.

- EXPOSURE TO RF ENERGY

There has been some public concern about possible health effects of using GSM modem. Although research on health effects from RF energy has focused for many years on the current RF technology, scientists have begun research regarding newer radio technologies, such as GSM. After existing research had been reviewed, and after compliance to all applicable safety standards had been tested, it has been concluded that the product is fit for use.

If you are concerned about exposure to RF energy there are things you can do to minimize exposure. Obviously, limiting the duration of your calls will reduce your exposure to RF energy. In addition, you can reduce RF exposure by operating your cellular modem efficiently by following the below guidelines.

- EFFICIENT MODEM OPERATION

For your modem to operate at the lowest power level, consistent with satisfactory call quality :

If your modem has an extendible antenna, extend it fully. Some models allow you to place a call with the antenna retracted. However your modem operates more efficiently with the antenna fully extended.

Do not hold the antenna when the modem is « IN USE ». Holding the antenna affects call quality and may cause the modem to operate at a higher power level than needed.

- ANTENNA CARE AND REPLACEMENT

Do not use the modem with a damaged antenna. If a damaged antenna comes into contact with the skin, a minor burn may result. Replace a damaged antenna immediately. Consult your manual to see if you may change the antenna yourself. If so, use only a manufacturer-approved antenna. Otherwise, have your antenna repaired by a qualified technician.

Use only the supplied or approved antenna. Unauthorized antennas, modifications or attachments could damage the modem and may contravene local RF emission regulations or invalidate type approval.

## GENERAL SAFETY

- DRIVING

Check the laws and regulations on the use of cellular devices in the area where you drive. Always obey them. Also, when using your modem while driving, please :

- give full attention to driving,

- pull off the road and park before making or answering a call if driving conditions so require.

- ELECTRONIC DEVICES

Most electronic equipment, for example in hospitals and motor vehicles is shielded from RF energy. However RF energy may affect some malfunctioning or improperly shielded electronic equipment.

- VEHICLE ELECTRONIC EQUIPMENT

Check your vehicle manufacturer’s representative to determine if any on board electronic equipment is adequately shielded from RF energy.

- MEDICAL ELECTRONIC EQUIPMENT

Consult the manufacturer of any personal medical devices (such as pacemakers, hearing aids, etc...) to determine if they are adequately shielded from external RF energy.

Turn your modem OFF in health care facilities when any regulations posted in the area instruct you to do so. Hospitals or health care facilities may be using RF monitoring equipment.

- AIRCRAFT

Turn your modem OFF before boarding any aircraft.

- Use it on the ground only with crew permission.

- Do not use in the air.

To prevent possible interference with aircraft systems, Federal Aviation Administration (FAA) regulations require you to have permission from a crew member to use your modem while the plane is on the ground. To prevent interference with cellular systems, local RF regulations prohibit using your modem whilst airborne.

- CHILDREN

Do not allow children to play with your modem. It is not a toy. Children could hurt themselves or others (by poking themselves or others in the eye with the antenna, for example). Children could damage the modem, or make calls that increase your modem bills.

- BLASTING AREAS

To avoid interfering with blasting operations, turn your unit OFF when in a « blasting area » or in areas posted : « turn off two-way radio ». Construction crew often use remote control RF devices to set off explosives.

- POTENTIALLY EXPLOSIVE ATMOSPHERES

Turn your modem OFF when in any area with a potentially explosive atmosphere. It is rare, but your modem or its accessories could generate sparks. Sparks in such areas could  cause an explosion or fire resulting in bidily injury or even death.

Areas with a potentially explosive atmosphere are often, but not always, clearly marked. They include fueling areas such as petrol stations ; below decks on boats ; fuel or chemical transfer or storage facilities ; and areas where the air contains chemicals or particles, such as grain, dust, or metal powders.

Do not transport or store flammable gas, liquid, or explosives, in the compartment of your vehicle which contains your modem or accessories.

Before using your modem in a vehicle powered by liquefied petroleum gas (such as propane or butane) ensure that the vehicle complies with the relevant fire and safety regulations of the country in which the vehicle is to be used.

## SAFETY STANDARDS

THIS CELLULAR MODEM COMPLIES WITH ALL APPLICABLE RF SAFETY STANDARDS.

This cellular modem meets the standards and recommendations for the protection of public exposure to RF electromagnetic energy established by governmental bodies and other qualified organizations, such as the following :

Directives of the European Community, Directorate General V in Matters of Radio         Frequency Electromagnetic Energy