### AlphaServer 800 Pedestal and Rackmount
V97-1.2—1 May 1997
DIGITAL Systems and Options Catalog

Product Description	The AlphaServer 800 is an Alpha microprocessor server, available in 333 MHz and 400 MHz with 2 MB ECC cache.  The system offers up to 8 internal storage devices including a floppy diskette drive, three 5.25-inch bays for optional removable media drives, and four hot swap storage disks.  The system supports up to 1 GB of memory and up to 36 GB of internal storage with four 9 GB disks. Integrated on  the system board are a Fast Wide Single-Ended SCSI-2 controller, SVGA controller with 1 MB Video ram,  a Remote Management Console,  two serial ports, one parallel port, and keyboard and mouse interfaces. AlphaServer 800 is available with DIGITAL UNIX, OpenVMS, or  Microsoft Windows NT.

The AlphaServer 800 is available in two packages—Pedestal and Rackmount.  The pedestal package has an extremely small footprint and can be located practically anywhere in an office environment. The Rackmount package can be mounted in any cabinet equipped with industry-standard RETMA or METRIC rails in compliance with standard EIA 310D and occupies only 8.75 inches  [5U] of vertical rack space.

The AlphaServer 800 offers the following high-reliability features: temperature sensor, fan speed sensor, power supply over/under protection and ECC memory. The AlphaServer 800 integrated Remote Management Console (RMC) features significantly enhances the ability to remotely manage and diagnose system conditions. The RMC allows a remote operator to monitor server voltages, temperatures, and fan, as well as manipulate the server (Power On/Off, reset, halt).  The RMC is powered by an auxiliary regulator in the main power system that allows system to be remotely interrogated even if system is turned off or shuts down.  In the event of an alert condition, the RMC can be setup to automatically dial a user definable pager phone or another system to make a remote operator aware of the alert condition.

Advanced server management features are provided with all AlphaServer 800 shipments via the bundled ServerWORKS Manager kit.  The kit provides remote management capability through Simple Network Management Protocol agents (SNMP) for Windows NT, DIGITAL UNIX. and OpenVMS.  The  management console software runs on any Windows NT or Windows 95 PC. The SNMP agents required to manage the server ship with the TCP/IP services of the operating system (with the exception of Windows NT which ships with the ServerWORKS kit).

The SNMP agents collect critical server information including system status, I/O information, storage and disk information and network status information.  Thresholds can be set to monitor key server events and alarms are forwarded to the ServerWORKS console.  ServerWORKS  Manager V2.0 includes remote pager support for alarm notification.  In addition to ServerWORKS, DIGITAL includes key third party software applications including DIGITAL Power Management Software for Alpha provided  by EXIDE, and application/database management software through BMC software's PATROL family of products.

DIGITAL believes the information in this publication is accurate as of its publication date; such information is subject to change without notice. DIGITAL is not responsible for any inadvertent errors.

DIGITAL conducts its business in a manner that conserves the environment and protects the safety and health of its employees, customers, and the community.

DIGITAL, the DIGITAL logo are trademarks of Digital Equipment Corporation.

Printed in USA. Copyright 1997Digital Equipment Corporation. All rights reserved.

### Step 1—AlphaServer 800 Pedestal and Rackmount System

- Windows NT Packaged systems include factory installed (FIS) Windows NT Server 4.0 on hard disk drive.

- North American variants include Windows NT license, media (CD-ROM) kit North American English

- Non-North American variants include factory installed (FIS) Windows NT 4.0 English.

- Selection of language specific Windows NT license, media (CD-ROM) kit is mandatory for all non-North American variants, see Step 7.

- DIGITAL UNIX and OpenVMS Packaged systems include factory installed (FIS) software on hard disk drive.

- All DIGITAL UNIX systems ship with UNIX V4.0B

- All DIGITAL OpenVMS systems ship with OpenVMS V7.1.

- Rackmount systems include slide rails and mounting hardware to mount system in any cabinet equipped with industry-standard RETMA or METRIC rails in compliance with standard EIA 310D; see Step 9 for  recommended DIGITAL cabinets.

- Uninterruptable Power Supplies are available; see UPS Information following System Specifications.

- Options ordered  that are factory installable, will be factory installed unless specified as spares.

Pedestal and Rackmount Systems include

- Alpha microprocessor 21164

- 333 MHz CPU with 2 MB ECC onboard cache, or

- 400-MHz CPU with 2 MB ECC onboard cache

- Pedestal and Rackmount enclosures with:

- Six Bus Slots
Three 32-bit PCI slots
Two EISA slots
One shared 64-bit PCI and EISA slot

- 8 DIMM memory slots support 2 memory options

- 8 storage slots:
One diskette drive slot
Three removable media slots
Four hot swap storage disks wide and/or UltraWide SCSI drive slots

- Integral Fast Wide Single-Ended SCSI-2 controller

- Integrated SVGA with 1 MB Video Ram

- 300-Watt switch select (120/240V, 60/50Hz) power supply

- Two serial ports, support full duplex asynchronous modem control

- One parallel port

- PS/2 style keyboard port and mouse port

- PCI-based Fast 100 Ethernet (Twisted Pair)—uses one PCI slot

- 600 MB 12X CD-ROM—uses one removable media  slot

- One 2.1 GB wide hard disk drive

- 1.44 MB diskette drive in dedicated slot

- One memory option

- North American Windows NT and DIGITAL UNIX variants include a 104-key, PC style U.S. English keyboard.

- North American OpenVMS variants include a 108 key, VT style U.S. English keyboard.

- Select country-specific keyboard for all non-U.S. variants, see Step 8.

- 3-button mouse

- North American variants include 120V power cord (North America, Japan).  Mandatory selection of country-specific power cord for all non-North American variants, see Step 8.

- English language documentation

- EISA Configuration Utility (ECU)

- Integrated Advanced Server Management features, including ServerWorks Manager kit

- Hardware Warranty, Three-year on-site*

- Software Warranty*

- DIGITAL UNIX and OpenVMS, 90-day SPD conformance with advisory telephone support

- Warranty on Microsoft's Windows NT Server 4.0 is conformance to the written material accompanying the software for a period of ninety days

- Windows NT Server 4.0 plus 10-client access license, media (CD-ROM) kit, or†

- DIGITAL UNIX  V4.0B, Unlimited User license, Server Extension license, Internet AlphaServer System Software kit, or

- OpenVMS V7.1 base license, System Manager license, Enterprise Integrated Package (EIP)

*	Service upgrades are available; see Step 10, Hardware and Software Supplemental Services

†	Windows NT language specific media kit is mandatory for non-North American variants; see Step 7.

### Step 1—AlphaServer 800 5/333 Pedestal and Rackmount Systems

| AlphaServer 800 5/333 Pedestal Packaged Systems |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Order Number | Operating System | O/S Media | Power cord | Keyboard | Memory | Disk | Graphics | Monitor |
| PB80B-AA | Windows NT | FIS | 120 V | Included | 64 MB | 2.1 GB | Included | Required |
| PB80B-AB | Windows NT | Mandatory* | Mandatory | Required | 64 MB | 2.1 GB | Included | Required |
| PB80B-AC | Windows NT | FIS | 120 V | Included | 128 MB | 2.1 GB | Included | Required |
| PB80B-AD | Windows NT | Mandatory* | Mandatory | Required | 128 MB | 2.1 GB | Included | Required |
| PB80B-FA | DIGITAL UNIX | FIS | 120 V | Included | 64 MB | 2.1 GB | Included | Recommended |
| PB80B-FB | DIGITAL UNIX | FIS | Mandatory | Required | 64 MB | 2.1 GB | Included | Recommended |
| PB80B-FC | DIGITAL UNIX | FIS | 120 V | Included | 128 MB | 2.1 GB | Included | Recommended |
| PB80B-FD | DIGITAL UNIX | FIS | Mandatory | Required | 128 MB | 2.1 GB | Included | Recommended |
| PB80B-MA | Open VMS | FIS | 120 V | Included | 64 MB | 2.1 GB | Included | Recommended |
| PB80B-MB | Open VMS | FIS | Mandatory | Required | 64 MB | 2.1 GB | Included | Recommended |
| PB80B-MC | Open VMS | FIS | 120 V | Included | 128 MB | 2.1 GB | Included | Recommended |
| PB80B-MD | Open VMS | FIS | Mandatory | Required | 128 MB | 2.1 GB | Included | Recommended |

| AlphaServer 800 5/333 Rackmount Packaged Systems |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Order Number | Operating System | O/S Media | Power cord | Keyboard | Memory | Disk | Graphics | Monitor |
| PB80P-AA | Windows NT | FIS | 120 V | Included | 64 MB | 2.1 GB | Included | Required |
| PB80P-AB | Windows NT | Mandatory* | Mandatory | Required | 64 MB | 2.1 GB | Included | Required |
| PB80P-AC | Windows NT | FIS | 120 V | Included | 128 MB | 2.1 GB | Included | Required |
| PB80P-AD | Windows NT | Mandatory* | Mandatory | Required | 128 MB | 2.1 GB | Included | Required |
| PB80P-FA | DIGITAL UNIX | FIS | 120 V | Included | 64 MB | 2.1 GB | Included | Recommended |
| PB80P-FB | DIGITAL UNIX | FIS | Mandatory | Recommended | 64 MB | 2.1 GB | Included | Recommended |
| PB80P-FC | DIGITAL UNIX | FIS | 120 V | Included | 128 MB | 2.1 GB | Included | Recommended |
| PB80P-FD | DIGITAL UNIX | FIS | Mandatory | Recommended | 128 MB | 2.1 GB | Included | Recommended |
| PB80P-MA | Open VMS | FIS | 120 V | Included | 64 MB | 2.1 GB | Included | Recommended |
| PB80P-MB | Open VMS | FIS | Mandatory | Recommended | 64 MB | 2.1 GB | Included | Recommended |
| PB80P-MC | Open VMS | FIS | 120 V | Included | 128 MB | 2.1 GB | Included | Recommended |
| PB80P-MD | OpenVMS | FIS | Mandatory | Recommended | 128 MB | 2.1 GB | Included | Recommended |

*	Includes Factory Installed Windows NT Server 4.0, English.  Selection of Windows NT Server license and language specific media kit is mandatory.

Key:	Mandatory items must be on purchase order at initial order acceptance

Required items are essential for full system operation

Recommended items enhance system functionality

FIS = Factory Installed Software

### Step 1—AlphaServer 800 5/400 Pedestal and Rackmount Systems

| AlphaServer 5/400 Pedestal Packaged Systems |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Order Number | Operating System | O/S Media | Power cord | Keyboard | Memory | Disk | Graphics | Monitor |
| PB81B-AA | Windows NT | FIS | 120 V | Included | 64 MB | 2.1 GB | Included | Required |
| PB81B-AB | Windows NT | Mandatory* | Mandatory | Required | 64 MB | 2.1 GB | Included | Required |
| PB81B-AC | Windows NT | FIS | 120 V | Included | 128 MB | 2.1 GB | Included | Required |
| PB81B-AD | Windows NT | Mandatory* | Mandatory | Required | 128 MB | 2.1 GB | Included | Required |
| PB81B-AE | Windows NT | FIS | 120 V | Included | 256 MB | 2.1 GB | Included | Required |
| PB81B-AF | Windows NT | Mandatory* | Mandatory | Required | 256 MB | 2.1 GB | Included | Required |
| PB81B-FA | DIGITAL UNIX | FIS | 120 V | Included | 64 MB | 2.1 GB | Included | Recommended |
| PB81B-FB | DIGITAL UNIX | FIS | Mandatory | Recommended | 64 MB | 2.1 GB | Included | Recommended |
| PB81B-FC | DIGITAL UNIX | FIS | 120 V | Included | 128 MB | 2.1 GB | Included | Recommended |
| PB81B-FD | DIGITAL UNIX | FIS | Mandatory | Recommended | 128 MB | 2.1 GB | Included | Recommended |
| PB81B-FE | DIGITAL UNIX | FIS | 120 V | Included | 256 MB | 2.1 GB | Included | Recommended |
| PB81B-FF | DIGITAL UNIX | FIS | Mandatory | Recommended | 256 MB | 2.1 GB | Included | Recommended |
| PB81B-MA | Open VMS | FIS | 120 V | Included | 64 MB | 2.1 GB | Included | Recommended |
| PB81B-MB | Open VMS | FIS | Mandatory | Recommended | 64 MB | 2.1 GB | Included | Recommended |
| PB81B-MC | Open VMS | FIS | 120 V | Included | 128 MB | 2.1 GB | Included | Recommended |
| PB81B-MD | Open VMS | FIS | Mandatory | Recommended | 128 MB | 2.1 GB | Included | Recommended |
| PB81B-ME | Open VMS | FIS | 120 V | Included | 256 MB | 2.1 GB | Included | Recommended |
| PB81B-MF | Open VMS | FIS | Mandatory | Recommended | 256 MB | 2.1 GB | Included | Recommended |

| AlphaServer 800 5/400 Rackmount Packaged systems |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Order Number | Operating System | O/S Media | Power cord | Keyboard | Memory | Disk | Graphics | Monitor |
| PB81P-AA | Windows NT | FIS | 120 V | Included | 64 MB | 2.1 GB | Included | Required |
| PB81P-AB | Windows NT | Mandatory* | Mandatory | Required | 64 MB | 2.1 GB | Included | Required |
| PB81P-AC | Windows NT | FIS | 120 V | Included | 128 MB | 2.1 GB | Included | Required |
| PB81P-AD | Windows NT | Mandatory* | Mandatory | Required | 128 MB | 2.1 GB | Included | Required |
| PB81P-AE | Windows NT | FIS | 120 V | Included | 256 MB | 2.1 GB | Included | Required |
| PB81P-AF | Windows NT | Mandatory* | Mandatory | Required | 256 MB | 2.1 GB | Included | Required |
| PB81P-FA | DIGITAL UNIX | FIS | 120 V | Included | 64 MB | 2.1 GB | Included | Recommended |
| PB81P-FB | DIGITAL UNIX | FIS | Mandatory | Recommended | 64 MB | 2.1 GB | Included | Recommended |
| PB81P-FC | DIGITAL UNIX | FIS | 120 V | Included | 128 MB | 2.1 GB | Included | Recommended |
| PB81P-FD | DIGITAL UNIX | FIS | Mandatory | Recommended | 128 MB | 2.1 GB | Included | Recommended |
| PB81P-FE | DIGITAL UNIX | FIS | 120 V | Included | 256 MB | 2.1 GB | Included | Recommended |
| PB81P-FF | DIGITAL UNIX | FIS | Mandatory | Recommended | 256 MB | 2.1 GB | Included | Recommended |
| PB81P-MA | Open VMS | FIS | 120 V | Included | 64 MB | 2.1 GB | Included | Recommended |
| PB81P-MB | Open VMS | FIS | Mandatory | Recommended | 64 MB | 2.1 GB | Included | Recommended |
| PB81P-MC | Open VMS | FIS | 120 V | Included | 128 MB | 2.1 GB | Included | Recommended |
| PB81P-MD | Open VMS | FIS | Mandatory | Recommended | 128 MB | 2.1 GB | Included | Recommended |
| PB81P-ME | Open VMS | FIS | 120 V | Included | 256 MB | 2.1 GB | Included | Recommended |
| PB81P-MF | Open VMS | FIS | Mandatory | Recommended | 256 MB | 2.1 GB | Included | Recommended |

*	Includes Factory Installed Windows NT Server 4.0, English.  Selection of Windows NT Server license and language specific media kit is mandatory.

Key:	Mandatory items must be on purchase order at initial order acceptance

Required items are essential for full system operation

Recommended items enhance system functionality

FIS = Factory Installed Software

Step 2—Memory

- Packaged Systems include one 64 MB, 128 MB or 256 MB memory option.

- System supports one addition memory option for a maximum of two memory options supported  per AlphaServer 800 system.  Memory options include industry-standard EDO DIMMs with ECC support.

- System maximum of 1 GB memory

| PB8MA-AB | –32 MB 70ns (4 x ––8 MB DIMMs) memory option |
| --- | --- |
| PB8MA-AC | –64 MB 70ns (4 x –16 MB DIMMs) memory option |
| PB8MA-AD | 128 MB 70ns (4 x –32 MB DIMMs) memory option |
| PB8MA-AE | 256 MB 70ns (4 x –64 MB DIMMs) memory option |
| PB8MA-AF | 512 MB 70ns (4 x 128 MB DIMMs) memory option |

Step 2a—Prestoserve Non-Volatile Random Access Memory (NVRAM)

- Supported on DIGITAL UNIX systems only, maximum one Prestoserve option per system.

| DJ-ML200-BA | PCI-based 4 MB PrestoServe I/O performance enhancement option |
| --- | --- |

Step 3—Monitors

- Windows NT systems require a graphics monitor to run all functions.

- Graphics monitors other than those listed below can be used if compatible with SVGA graphics included with system.

Note: Higher resolution available with optional EISA or PCI graphics adapters (see Step 5).

| SN-VRCX5-WA/W3/W4 | 15" (13.9" viewable image size) Corporate Series auto-scan color monitor, flat square invar CRT, 0.28 mm dot pitch, VGA to 1024 x 768 @ 85 Hz, TCO 92, MPRII, Energy Star, attached video cable. -WA = Northern Hemisphere with 120 V power cord, -W3 = Northern Hemisphere without power cord. Select country specific power cords for -W3 Northern Hemisphere, and -W4 Southern Hemisphere variants. |
| --- | --- |

| VRTX7-WA/W3 VRT17-W4 | 17” (16.0” viewable image size) professional series auto-scanning color monitor, Trinitron CRT, 0.25 mm aperture grill pitch, VGA to 1280 x 1024 at 75 Hz, TCO 92, MPR-II, Energy Star, HD15 male to male video cable. -WA = Northern Hemisphere with 120V power cord. -W3 = Northern Hemisphere without power cord. -W4 = Southern Hemisphere without power cord. Select country specific power cord for -W3 and -W4 variants. |
| --- | --- |

| VRCX1-WA/W3/W4 | 21" (19.7" viewable image size) auto-scanning color monitor, 0.28 Dot Pitch, VGA to 1600 x 1200 at 75 Hz NI, TCO 92, Energy Star, includes video cable. -WA = Northern Hemisphere with 120 V power cord. -W3 = Northern Hemisphere without power cord. W4 = Southern Hemisphere without power cord. Select country specific power cord for W3 and W4 variants. |
| --- | --- |

Step 4—Storage

Configuration rules for Pedestal and Rackmount Storage

- Integral Fast Wide Single Ended (FWSE) SCSI-2 controller supports a maximum of seven wide devices.

- Three 5.25 inch half-height removable media devices or One 5.25 inch half-height and one 5.25 full-height removable media devices.

- Four internal hot-swappable SCSI SCA2 hard drives

Step 4a—Internal Storage

- Pedestal and Rackmount Packaged systems include:

- One CD-ROM drive

- One 2.1 GB SCSI SCA2 wide hard drive

- 1.44 MB diskette drive in dedicated slot.

Step 4a—Internal Storage (continued)

Removable Media Devices

| RRD46-AB | 600 MB 5.25 inch half-height 12X CD-ROM drive |
| --- | --- |
| PBXTZ-AA | 2.0 GB 5.25 inch half-height SCSI QIC tape drive (TZK11) |
| TLZ09-LK | 8.0 GB 5.25 inch x 1.6-inch SCSI 4 mm DAT drive |
| TLZ9L-LG | 32/64 GB 5.25 inch full-height DAT loader |

System Hard Disk Drives

Pedestal and Rackmount Packaged systems include one 2.1 GB SCSI SCA2 hard disk drive (RZ28M-SB)

| RZ26N-SB | 1.05 GB 16-bit wide 5400 RPM 3.5 x 1” SCSI SCA2 hard disk drive |
| --- | --- |
| RZ28M-SB | 2.1 GB 16-bit wide 5400 RPM 3.5 x 1” SCSI SCA2 hard disk drive |
| RZ1BB-SB | 2.1 GB UltraWide SCSI SCA2 7200 RPM 3.5 x 1” hard disk drive |
| RZ1CB-SB | 4.3 GB UltraWide SCSI SCA2 7200 RPM 3.5 x 1” hard disk drive |
| RZ1DB-SB | 9.0 GB UltraWide SCSI SCA2 7200 RPM 3.5 x 1.6” hard disk drive |

Internal SCSI Cable Kits

| PB8HA-DA | Includes 68-pin SCSI cable, connects 16-bit wide SCSI controller to internal storage assembly Example—connects KZPAC-xx PCI RAID controller to internal AlphaServer 800 storage assembly, maximum one per system. |
| --- | --- |
| BC25V-1H | Includes 68-pin SCSI cable, connects 16-bit wide SCSI Controller to bulkhead for external 16-bit wide SCSI expansion, maximum two per system |

Step 4b—Storage Controllers

- Maximum four PCI-based KZPAC-xx SCSI controllers supported per system.

- Maximum three EISA-based KZESC-xx SCSI controllers supported per system.  Maximum number of EISA-based controllers of all types is limited by the total number of available EISA slots and available IRQs.

- Maximum combination number of KZESC-xx and KZPAC-xx SCSI controllers supported is four.

- KZPAC-xx StorageWorks RAID Array 230/Plus includes PCI backplane RAID controller and StorageWorks RAID Array 230/Plus Subsystem family software and documentation kit for OpenVMS, DIGITAL UNIX and Windows NT.

- KZPAC-AA one-port controller installed in system requires a PB8HA-DA SCSI cable kit for connection to internal storage assembly.

- KZPAC-CA three-port controller installed in system requires one PB8HA-DA SCSI cable kit for connection to internal storage assembly , or one BC25V-1H cable kit for connection to external expansion port bulkhead.

- KZESC-xx StorageWorks RAID Array 200 includes EISA backplane RAID controller and StorageWorks RAID Array 200 Subsystem family software and documentation kit for OpenVMS, DIGITAL UNIX and Windows NT.

- KZESC-AA/BA controllers support external disks 
only.  Connection to the internal AlphaServer 800
Storage assembly is not supported.

- KZESC-BA three-port controller installed in system requires two BC25V-1H cable kit for connection to external expansion port bulkhead.

- KZPSA-BB PCI-based one-port Fast Wide Differential SCSI controller supports externally connected wide devices in BA356/BA36R with DWZZB signal converter, or narrow disks in BA35R with DWZZA signal converter.  Internal hard disk drives are not supported on FWD controller.

- SCSI cables are not included and must be ordered separately.

- External DSSI cables are not included and must be ordered separately.

SCSI Controllers

| KZPAA-AA | PCI-based one-port high-performance Fast Narrow Single Ended (FNSE) SCSI-2 controller |
| --- | --- |
| KZPSA-BB | PCI-based one-port high-performance Fast Wide Differential (FWD) SCSI controller. Supports external disks only. |
| BN21K-02 | Connects from KZPSA to DWZZA-VA in narrow StorageWorks enclosures and narrow StorageWorks Rackmount shelves, or DWZZB-VW in wide StorageWorks enclosures and wide StorageWorks Rackmount shelves. |

Step 4b—Storage Controllers (continued)

SCSI Controllers

| KZPDA-AA | PCI-based one-port Fast Wide Single Ended (FWSE) SCSI controller |
| --- | --- |
| BN21K-02 | Connects from KZPDA to wide StorageWorks enclosures and wide StorageWorks Rackmount shelves. |
| KZPAC-AA* | One-port PCI backplane RAID (FWSE) controller, UltraWide SCSI ready, with 4 MB EDRAM cache; includes StorageWorks RAID Array 230/Plus Subsystem family software and documentation kit. System requires PB8HA-DA SCSI cable kit for connection to internal Storage assembly. |
| PB8HA-DA | 68-pin SCSI cable assembly for connection to internal storage assembly, maximum one per system |
| BN31S-1E | Connects from KZPAC-AA to wide StorageWorks enclosures and wide StorageWorks Rackmount shelves |
| KZPAC-CA* | Three-port PCI backplane RAID (FWSE) controller, UltraWide SCSI ready, with 4 MB of EDRAM cache ; includes StorageWorks RAID Array 230/Plus Subsystem family software and documentation kit. System requires PB8HA-DA SCSI cable kit for connection to internal Storage assembly. |
| KZPAC-CB* | Save as above with 8 MB of EDRAM cache |
| PB8HA-DA | 68-pin SCSI cable assembly for connection to internal storage assembly, maximum one per system |
| BC25V-1H | SCSI cable assembly kit for connection to third port; uses one expansion bulkhead port, uses one PCI/ESIA bulkhead slot if three KZPAC-CA/CB are configured in system. |
| BN31S-1E | Connects from KZPAC-CA to wide StorageWorks enclosures and wide StorageWorks Rackmount shelves |
| KZPSC-UB | Battery backup for the KZPAC-xx. |
| KZESC-AA* | One-port EISA backplane RAID (FNSE) controller with 4 MB DRAM cache memory; includes StorageWorks RAID Array 200 Subsystem family software and documentation kit. |
| BN21H-02 | Connects from KZPAA-AA to narrow StorageWorks enclosures and narrow StorageWorks Rackmount shelves |
| BN21N-02 | Connects from KZESC-BA to wide StorageWorks enclosures and wide StorageWorks Rackmount shelves |
| KZESC-BA* | Three-port EISA backplane RAID (FNSE) controller with 4 MB DRAM cache memory ; includes StorageWorks RAID Array 200 Subsystem family software and documentation kit. |
| CK-SWXCR-AA | SCSI cable/bulkhead assembly kit for KZESC-BA, required for connecting second and third ports to bulkhead slot. Uses one PCI or EISA slot. |
| BN21H-02 | Connects from KZESC-BA to narrow StorageWorks enclosures and wide StorageWorks Rackmount shelves |
| BN21N-02 | Connects from KZESC-BA to wide StorageWorks enclosures and wide StorageWorks Rackmount shelves |

*	See Storage Devices for additional information on StorageWorks RAID Array 2x0 Subsystems and external StorageWorks expansion

DSSI Controllers

| KFPSA-AA | PCI-based single-DSSI controller (OpenVMS systems only). See Step 4f DSSI cables. |
| --- | --- |
| KFESB-AA | EISA-based single-DSSI controller (OpenVMS systems only). Uses one EISA slot if system is end node in OpenVMS cluster, or two slots if system is middle node in OpenVMS cluster. Maximum two (if end node in cluster) or maximum of one (if middle node in cluster). See Step 4d DSSI cables. |

Step 4c—External Tape Expansion

- External tape drives are also supported on optional PCI-based SCSI controllers.

- KZPAA-AA—maximum bus length including cable and tape device cannot exceed 3.0 meters.

- KZPSA-BB—maximum bus length including cable and tape device cannot exceed 25.0 meters.

- External tape drives are not supported on one-and three-port (KZESC-xx) Fast-SCSI-2 controllers.

- Each tabletop tape device requires high density 50-pin to low density 50-pin SCSI cable (BN23G-xx).

Step 4c—External Tape Expansion (continued)

External Tapes supported on Windows NT Servers

| TLZ09 | 4.0/8.0 GB 4 mm DAT tape drive |
| --- | --- |
| TLZ9L1,2 | 32/64 GB 4 mm DAT autoloader |
| TZK11-DA | 2.0 GB 5.25-inch tabletop QIC tape drive |
| TZ8752 | 100 GB, DLT tape autoloader |
| TZ8772 | 140 GB, DLT tape autoloader |
| BN23G-0E | 3 Foot Molded SCSI Cable, required for each tabletop tape device |

External Tapes supported on DIGITAL UNIX and OpenVMS servers

| TLZ09 | 4.0/8.0 GB 4 mm DAT tape drive |
| --- | --- |
| TLZ9L1,2, | 32/64 GB 4 mm DAT autoloader |
| TZK11-DA | 2.0 GB 5.25-inch tabletop QIC tape drive |
| TKZ60-FA/HA | 400 MB IBM 3480/3490 compatible tabletop tape drive |
| TKZ60-EA/GA | 400 MB IBM 3480/3490 compatible tabletop tape drive |
| TKZ61-AA/AD | 400 MB IBM compatible tape autoloader rackmountable |
| TKZ61-AC/AF | 400 MB IBM compatible tape autoloader tabletop |
| TKZ62-AA/AD | 2.4 GB IBM compatible tape autoloader rackmountable |
| TKZ62-AC/AF | 2.4 GB IBM compatible tape autoloader tabletop |
| TSZ07-CA/FA | 40/140 MB, reel/reel, tabletop tape drive |
| TSZ07-AA/DA | 40/140 MB, reel/reel, tape drive rackmountable |
| TSZ07-BA/EA | 40/140 MB, reel/reel, tape in a H9A10 cabinet |
| TZ8772 | 140 GB, DLT tape autoloader |
| TZ882 | 20/40 GB, DLT tape drive |
| TZ887-NT | 140/280 GB SCSI tape subsystem |
| TKZ9E-TA | 2/5/7/10/14 GB 8 mm helical scan tape drive, tabletop |
| TKZ9E-VA | 2/5/7/10/14 GB 8 mm helical scan tape drive in StorageWorks SBB carrier |
| BN23G-01 | 3 Foot Molded SCSI Cable, required for each tabletop tape device. |

1		Includes four cartridge loader. Larger magazines are supported.

2	Base operating systems support sequential back-up mode only; additional software is required for random access backup. See Storage Devices for details.

Step 4d—DSSI Cables

- KFESB-AA EISA-based DSSI controller cables:

- KFESB to any external “Pin-Socket” DSSI connection (VAX 4000s, R400X) requires BC22Q-xx DSSI cable.

- KFESB to any external “Micro-Ribbon” DSSI straight connection (all other DSSI systems and storage devices requiring straight connection) requires BC21Q-xx DSSI cable.

- KFESB to any external “Micro-Ribbon” DSSI right-angle connection (all other DSSI systems and storage devices requiring right-angle connection) requires BC29S-xx DSSI cable.

- KFPSA PCI-based DSSI controller cables:

- KFPSA to any external “Pin-Socket” DSSI connection requires BC22Q-xx

- KFPSA to any external “Micro-Ribbon” DSSI straight connection (all other DSSI systems and storage devices requiring straight connection) requires BC21Q-xx

- KFPSA to any external “Micro-Ribbon” DSSI right-angle connection (all other DSSI systems and storage devices requiring right-angle connection) requires BC29S-xx DSSI cable

- Order BC29S-09 DSSI cable for HSD10 in BA36R-Ax shelves

- Order BC29U-02 DSSI cable for HSD10 in adjacent BA36R-Ax shelves

- Order BC29V-06 DSSI cable for HSD10 in non-adjacent BA36R-Ax shelves

Step 5—Graphics Adapters

Note:	Select only if additional graphics card is required.

- All systems include onboard SVGA with 1 MB Video Ram.

- Maximum of one PB2GA-JD supported per system.  Onboard SVGA must be disabled.

- Maximum four PBXGB-AA/CA supported on DIGITAL UNIX systems
Maximum one PBXGB-AA/CA supported on OpenVMS and Windows NT. Onboard SVGA must be disabled.

| PB2GA-JD | PCI-based 2-MB DRAM graphics adapter (DIGITAL UNIX, OpenVMS, Windows NT) |
| --- | --- |
| PBXGB-AA | PCI-based 8-plane 1280 x 1024 graphics adapter (DIGITAL UNIX. OpenVMS, Windows NT) |
| PBXGB-CA | PCI-based 24-plane 1280 x 1024 x 1024 graphics adapter (Digital UNIX, OpenVMS. Windows NT) |

Step 6—Networks and Communications

- Packaged systems include PCI-based Fast 100 Ethernet (Twisted Pair) controller (DE500-AA), uses one PCI slot.

- Select networking cable for Ethernet controller

- BN25G-03/04/07 for 10BaseT (twisted pair)

- Maximum number of each EISA-based network controllers supported per system:

- Three DE425-AA, Three DW300-AA, Three DNSES

| DE500-AA | PCI-based Fast Ethernet network interface card |
| --- | --- |
| DE450-CA | PCI-based DIGITAL Etherworks 32-bit network interface card (twisted pair, Thick wire, ThinWire) |
| DEFPA-AB | PCI-based DEC FDDIcontroller, Single Attachment; maximum four, OpenVMS, Windows NT, DIGITAL UNIX |
| DEFPA-DB | PCI-based DEC FDDIcontroller, Dual Attachment, maximum four, OpenVMS, Windows NT, DIGITAL UNIX |
| DEFPA-UB | PCI-based DEC FDDI (UTP) controller; maximum four, OpenVMS, Windows NT, DIGITAL UNIX |
| DGLPB-AB | PCI-based ATMworks 350 adapter, DIGITAL UNIX only |
| PBXDP-AA | PCI-based , 2-port, synchronous communications controller, maximum four, OpenVMS, Windows NT |
| PBXDP-AB | PCI-based , 4-port, synchronous communications controller, maximum four, OpenVMS, Windows NT |
| PBXDP-AC | PCI-based , 8-port, synchronous communications controller, maximum four, OpenVMS, Windows NT |
| PBXNP-AA | PCI-based Token-Ring, DIGITAL UNIX, OpenVMS |
| PBXDA-AA | PCI-based Async MUX Adapter, 4 lines, DIGITAL UNIX, Windows NT |
| PBXDA-AB | PCI-based Async MUX Adpater, 8 lines, DIGITAL UNIX, Windows NT |

| DE425-AA | EISA-based Ethernet, OpenVMS and DIGITAL UNIX only |
| --- | --- |
| DW300-AA | EISA-based Token-Ring adapter includes NetWare V2.15 driver, LAN Manager Driver, and documentation (Not supported by DECnet/OSI for OpenVMS) |
| DNSES-AA | EISA-based synchronous communications controller; DIGITAL UNIX, OpenVMS |

| CXI01-AA | ISA Async MUX Adapter, 16 lines¾expandable to 64 lines; DIGITAL UNIX, Windows NT |
| --- | --- |
| CXI01-AD | ISA Async MUX Adapter, 16 lines¾expandable to 224 lines; DIGITAL UNIX, Windows NT |
| CXI01-AB | Asynchronous 16 Port XEM expansion for CXI01-AA/AD |
| CXI01-AC | Asynchronous MUX cable converter (RJ45-DB25) |
| CXI01-AE | Asynchronous 16 Port EPC expansion for CXI01-AD, Maximum supported is four |
| CXI01-AF | Asynchronous MUX adapter (RJ45-MJ11), 8 pack |
| DIIAA-AA | ISA-based ISDN terminal adapter (U.S.), maximum one, Windows NT |
| DIIAA-AB | ISA-based ISDN terminal adapter (non-U.S.), maximum one, Windows NT |

Step 6—Networks and Communications (continued)

Video and Sound Options

| AV301-AA | PCI-based, Full Video Supreme, maximum one, DIGITAL UNIX, OpenVMS |
| --- | --- |
| AV301-AN | PCI-based, Full Video Supreme, maximum one, Windows NT |
| AV321-AA | PCI-based, Full Video Supreme JPEG, maximum one, DIGITAL UNIX, OpenVMS, Windows NT |
| AVA01-AA | ISA-based, Sound Card, head phones and microphone, maximum one, DIGITAL UNIX, OpenVMS, Windows NT |

Step 7—Software

- North American variants of Windows NT Packaged systems include Windows NT license, media (CD-ROM) kits (see Step 1).  Note:  Selection of language specific Windows NT media kit is mandatory for non North American variants.

Windows NT Server plus 10-client access license, media (CD-ROM) kits

| QB-23CAA-SB | Windows NT Server license, media kit North American English |
| --- | --- |
| QB-23C8A-SB | Windows NT Server license, media kit International English |
| QB-23CPA-SB | Windows NT Server license, media kit French |
| QB-23CGA-SB | Windows NT Server license, media kit German |
| QB-23CSA-SB | Windows NT Server license, media kit Spanish |
| QB-23CUA-SB | Windows NT Server license, media kit Italian |
| QB-23CJA-SB | Windows NT Server license, media kit Japanese |
| QB-23CTA-SB | Windows NT Server license, media kit Hebrew |
| QB-23CMA-SB | Windows NT Server license, media kit Swedish |
| QB-23CQA-SB | Windows NT Server license, media kit Arabic |
| QB-23C5A-SB | Windows NT Server license, media kit Thai |
| QB-23CHA-SB | Windows NT Server license, media kit Dutch |
| QB-23CVA-SB | Windows NT Server license, media kit Brazilian/Portuguese |
| QB-23C4A-SB | Windows NT Server license, media kit Korean |
| QB-23C3A-SB | Windows NT Server license, media kit Taiwanese |
| QB-23C2A-SB | Windows NT Server license, media kit PRC Chinese |

Windows NT Server Optional software and documentation

| QB-53V9A-SA | Windows NT Server Cluster Kit |
| --- | --- |

DIGITAL UNIX

Software Processor Code = E

- DIGITAL UNIX Packaged systems include Traditional Unlimited User license.

- DIGITAL UNIX Packaged systems require operating system media and documentation for first system on site.

DIGITAL UNIX Media and Documentation—required for first system on site

| QA-MT4AA-H8 | DIGITAL UNIX media and on-line documentation on CD-ROM |
| --- | --- |
| QA-MT4AA-GZ | DIGITAL UNIX full hardcopy documentation |

DIGITAL UNIX Layered Products CD-ROM

| QA-054AA-H8 | Layered products media and documentation for DIGITAL UNIX on CD-ROM |
| --- | --- |

DIGITAL UNIX ASE

| QL-05SAE-AA | DIGITAL UNIX Cluster license |
| --- | --- |

Step 7—Software

DECnet for DIGITAL UNIX

| QL-MTJAE-AA | DECnet/OSI end-system license for DIGITAL UNIX |
| --- | --- |
| QL-MTKAE-AA | DECnet/OSI extended function license for DIGITAL UNIX |

OpenVMS

Software Processor Code = E

OpenVMS Concurrent Use Licenses are not specific to a single system and can be moved between systems at user discretion. OpenVMS Concurrent Use Licenses can also be shared in a mixed OpenVMS VAX and OpenVMS Alpha Cluster.

OpenVMS Concurrent Use Licenses

| QL-MT3AA-3B | OpenVMS Concurrent Use 1-user license |
| --- | --- |
| QL-MT3AA-3C | OpenVMS Concurrent Use 2-user license |
| QL-MT3AA-3D | OpenVMS Concurrent Use 4-user license |
| QL-MT3AA-3E | OpenVMS Concurrent Use 8-user license |
| QL-MT3AA-3F | OpenVMS Concurrent Use 16-user license |
| QL-MT3AA-3G | OpenVMS Concurrent Use 32-user license |
| QL-MT3AA-3H | OpenVMS Concurrent Use 64-user license |
| QL-MT3AA-3J | OpenVMS Concurrent Use 128-user license |
| QL-MT3AA-3K | OpenVMS Concurrent Use 256-user license |
| QL-MT2AE-AA | OpenVMS Traditional unlimited user license |

OpenVMS Media and Documentation - required for first system on site

| QA-MT1AA-H8 | OpenVMS media and on-line documentation CD-ROM |
| --- | --- |
| QA-001AA-GZ | OpenVMS hardcopy documentation |

OpenVMS Layered Products CD-ROM

| QA-03XAA-H8* | Layered products media and documentation for OpenVMS on CD-ROM |
| --- | --- |

*	Includes DIGITAL Enteprise Integration Server for OpenVMS media and documentation

DIGITAL Enterprise Integration Package

| QA-5LVAA-H8 | DIGITAL Enterprise Integration Server for OpenVMS media and documentation |
| --- | --- |

DECnet for OpenVMS

| QL-MTGAE-AA | DECnet extended function license for OpenVMS |
| --- | --- |
| QL-MTHAE-AA | DECnet end-system to extended function upgrade license for OpenVMS |

DSSI Information

| EK-410AB-MG | DSSI VMScluster Installation Guide |
| --- | --- |
| EK-D4AXP-TS | DSSI VMScluster Troubleshooting Guide |

Step 8—Power Cords, Keyboards, and Miscellaneous

Pedestal and Monitor Power Cords

- North American variants include BN27Y-1J 120 V (North American, Japan) power cord.  If another power cord is selected, both power cords ship with system.

- For non-North American system variants, selection of a power cord is mandatory.

| BN27Y-1J | North American, Japan, 120 V, 75-inches long |
| --- | --- |
| BN19H-2E | Australia, New Zealand, 2.5 meters long |
| BN19C-2E | Central Europe, 2.5 meters long |
| BN19A-2E | UK, Ireland, 2.5 meters long |
| BN19E-2E | Switzerland, 2.5 meters long |
| BN19K-2E | Denmark, 2.5 meters long |
| BN19M-2E | Italy, 2.5 meters long |
| BN19S-2E | Egypt, India, South Africa, 2.5 meters long |
| BN18L-2E | Israel, 2.5 meters long |

Rackmount Power Cords

- North American variants include BN20Z-4E 120 V (North American, Japan) power cord.  If another power cord is selected, both power cords ship with system.

- For non-North American system variants, selection of a power cord is mandatory.

Note:	AlphaServer 800 Pedestal system power cords are less then 15-feet long and are not supported in cabinet enclosures.

| BN20Q-4E | 15-feet 240 V, North American, 50Hz power cord connects system to power distribution unit located inside cabinet |
| --- | --- |
| BN20Z-4E | 15-feet 120 V, 60Hz power cord connects system to power distribution unit located inside cabinet |

Keyboards

- Some variants include a keyboard (see Step 1).  For these variants, if an additional keyboard is ordered, both keyboards ship with system.

| 104/105 key PC style keyboard |  | Language | 108 key VT style keyboard |  |
| --- | --- | --- | --- | --- |
| Windows NT | DIGITAL UNIX |  | OpenVMS | DIGITAL UNIX |
| LK97w-A2 | LK97w-A2 | U.S./English | LK46w-A2 | LK46w-A2 |
| LK97w-AB | LK97w-AB | Belgian | LK46w-AB | LK46w-AB |
| LK97w-AC | LK97w-AC | Canadian/French | LK46w-AC | LK46w-AC |
| LK97w-AD | LK97w-AD | Danish | LK46w-AD | LK46w-AD |
| LK97w-AE | LK97w-AE | United Kingdom | LK46w-A2 | LK46w-A2 |
| LK97w-AF | LK97w-AF | Finnish | LK46w-AF | LK46w-AF |
| LK97w-AG | LK97w-AG | German | LK46w-AG | LK46w-AG |
| LK97w-AH | LK97w-AH | Dutch | LK46w-AH | LK46w-AH |
| LK97w-AI | LK97w-AI | Italian | LK46w-AI | LK46w-AI |
| LK97w-AJ | LK97w-AJ | Japan |  |  |
|  |  | Swiss/French | LK46w-AK | LK46w-AK |
|  |  | Swiss/German | LK46w-AL | LK46w-AL |
| LK97w-AK | LK97w-AK | Swiss/Generic |  |  |
| LK97w-AF | LK97w-AF | Swedish | LK46w-AM | LK46w-AM |
| LK97w-AN | LK97w-AN | Norwegian | LK46w-AN | LK46w-AN |
| LK97w-AP | LK97w-AP | French | LK46w-AP | LK46w-AP |
|  |  | Canadian/English |  | LK46w-AQ |
| LK97w-AR | LK97w-AR | Lat. America |  |  |

Step 8—Power Cords, Keyboards, and Miscellaneous  (continued)

Keyboards

| 104/105 key PC style keyboard |  | Language | 108 key VT style keyboard |  |
| --- | --- | --- | --- | --- |
| Windows NT | DIGITAL UNIX |  | OpenVMS | DIGITAL UNIX |
| LK97w-AS | LK97w-AS | Spanish | LK46w-AS | LK46w-AS |
| LK97w-AV | LK97w-AV | Portuguese | LK46w-AV | LK46w-AV |
| LK97w-BH | LK97w-BH | Greek | LK46w-BH | LK46w-BH |
| LK97w-AT | LK97w-AT | Hebrew | LK46w-AT | LK46w-AT |
| LK97w-BI | LK97w-BI | Taiwanese |  |  |
| LK97w-BP | LK97w-BP | Polish | LK46w-BP | LK46w-BP |
| LK97w-BQ | LK97w-BQ | Hungarian | LK46w-BQ | LK46w-BQ |
| LK97w-CZ | LK97w-CZ | Slovak | LK46w-CZ | LK46w-CZ |
| LK97w-BY | LK97w-BY | Serbian |  |  |
| LK97w-CB | LK97w-CB | Thai |  |  |
| LK97w-CQ | LK97w-CQ | Icelandic |  |  |
| LK97w-BV | LK97w-BV | Czech | LK46w-BV | LK46w-BV |
| LK97w-BR | LK97w-BR | Arabic |  |  |
| LK97w-BU | LK97w-BU | Turkish Q |  |  |
| LK97w-BW | LK97w-BW | Turkish F | LK46w-BW | LK46w-BW |
| LK97w-BT | LK97w-BT | Cyrillic |  |  |

Air Filter Option

| PB8HA-CA | Pedestal Air filter, placed in front of fixed storage disk area. |
| --- | --- |

Pedestal to Rackmount Conversion kit

| PB8HA-BA | AlphaServer 800 Pedestal to Rackmount conversion kit. Includes bezel, slide rails and mounting hardware necessary for mounting system in any cabinet equipped with industry-standard RETMA or METRIC rails. Rackmount power cords must be ordered separately. |
| --- | --- |

Mouse and Extension Cable Kit

| PBXWS-WA | 3-button mouse (included with all systems, order as spare or replacement) |
| --- | --- |
| 2T-450KM-AA | Extension cable kit for VGA, PC style keyboard, and mouse, for use with Rackmount systems. |

Step 9—Cabinet Enclosure

Select cabinet enclosure for Packaged AlphaServer 800 Rackmount systems, if required.

Note:	For integration of an AlphaServer 800 Rackmount system into a DIGITAL Cabinet, call Computer Special Systems (CSS) 1-800-DIGITAL.

The color of cabinets listed is Digital 277 Blue-Gray.

- H9A10 19-inch EIA Cabinet Enclosure Dimensions

- Outside:  66.9-inches high, 23.62-inches wide, 33.8-inches deep

- Internal usable rackmountable space: 56-inches high, 19-inches wide, 30.8-inches deep

- An H9A10 cabinet with two H7600 power controllers can support a maximum of six AlphaServer 800 Rackmount systems.

- H9A15 19-inch EIA cabinet Enclosure Dimensions

- Outside: 78.7-inches high, 23.62-inches wide, 33.4-inches deep

- Internal usable rackmountable space: 68.25-inches high, 19-inches wide, 29.8-inches deep

- An H9A15 cabinet with two H7600 power controllers can supported a maximum of six AlphaServer 800 Rackmount systems.

Step 9—Cabinet Enclosure (continued)

- H9A11 19-inch EIA Cabinet Enclosure Dimensions

- Outside: 43.3-inches high, 33.8-inches wide, 30.8-inches deep

- Internal usable rackmountable space: 35-inches high, 19-inches wide, 30.8 inches

- An H9A11 cabinet with one H7600 power controller can support a maximum of three AlphaServer 800 Rackmount systems.

- Cabinet Power Plugs

- 120v Cabinets have one or two L5-30P plugs

- 240v Cabinets have one or two IEC pin and sleeve 316 P6 plugs

| H9A10-RL | 120V EIA Cabinet assembly, with two power controller, rear swing door |
| --- | --- |
| H9A10-RT | 240V EIA Cabinet assembly, with two power controller, rear swing door |
| H9A10-EJ | 120V EIA Cabinet assembly, with two power controller, front and rear swing door |
| H9A10-EK | 240V EIA Cabinet assembly, with two power controller, front and rear swing door |

| H9A15-RA | 120V EIA Cabinet assembly, with two power controllers, rear swing door |
| --- | --- |
| H9A15-RG | 240V EIA Cabinet assembly, with two power controllers, rear swing door |
| H9A15-RE | EIA Cabinet assembly, no power, rear swing door |

| H9A11-RA | 120V EIA Cabinet assembly, with one power controller, lift-off rear door |
| --- | --- |
| H9A11-RG | 240V EIA Cabinet assembly, with one power controller, lift-off rear door |

Step 10—Hardware and Software Supplemental Support Services

Hardware—Americas and Asia Pacific only

- Systems include three-year hardware warranty, on-site with 5 x 9, 24-hour response time.

- Select optional Hardware Supplemental Support Services if required.

- Select optional Hardware Installation.

| FM-MK4HR-36 | 5 x 9, 4-hour response time |
| --- | --- |
| FM-MK512-36 | 5 x 12, 4-hour response time |
| FM-MK616-36 | 6 x 16, 4-hour response time |
| FM-MK724-36 | 7 x 24, 4-hour response time |
| FM-MKXHW-60 | Years 1-5, next day, Onsite |
| FM-MK4HR-60 | Years 1-5, 5x9, 4-hour response time |
| FM-MK512-60 | Years 1-5, 5x12, 4-hour response time |
| FM-MK616-60 | Years 1-5, 6x16, 4-hour response time |
| FM-MK724-60 | Years 1-5, 7x24, 4-hour response time |
| FM-24INT-IN | Hardware Installation |

Software—Americas and Asia Pacific only

- DIGITAL UNIX and OpenVMS systems include 90-day Conformance to SPD and Telephone Advisory Support. Select optional Software Supplemental Support Services, if required.

- Warranty on Microsoft’s Windows NT Server 4.0 is conformance to the written material accompanying the software for a period of 90 days.

- Software service upgrades for Windows NT include advisory and remedial software support for the time period indicated.

| FM-WNT03-12 | 12-month Software Supplemental Support for Windows NT AlphaServer 800 systems |
| --- | --- |
| FM-WNT03-36 | 36-month Software Supplemental Support for Windows NT AlphaServer 800 systems |
| FM-WNT03-60 | 60-month Software Supplemental Support for Windows NT AlphaServer 800 systems |

Step 10—Hardware and Software Supplemental Support Services (continued)

Software—Americas and Asia Pacific only

| FM-80BUS-12 | 12-month Full Software Supplemental Support for DIGITAL UNIX AlphaServer 800 systems |
| --- | --- |
| FM-80BUS-36 | 36-month Full Software Supplemental Support for DIGITAL UNIX AlphaServer 800 systems |
| FM-80BUS-60 | 60-month Full Software Supplemental Support for DIGITAL UNIX AlphaServer 800 systems |
| FM-80BUN-12 | 12-month Node Software Supplemental Support for DIGITAL UNIX AlphaServer 800 systems |
| FM-80BUN-36 | 36-month Node Software Supplemental Support for DIGITAL UNIX AlphaServer 800 systems |
| FM-80BUN-60 | 60-month Node Software Supplemental Support for DIGITAL UNIX AlphaServer 800 systems |
| FM-80BVS-12 | 12-month Software Supplemental Support for OpenVMS AlphaServer 800 systems |
| FM-80BVS-36 | 36-month Software Supplemental Support for OpenVMS AlphaServer 800 systems |
| FM-80BVS-60 | 60-month Software Supplemental Support for OpenVMS AlphaServer 800 systems |
| FM-80BVN-12 | 12-month Node Software Supplemental Support for OpenVMS AlphaServer 800 systems |
| FM-80BVN-36 | 36-month Node Software Supplemental Support for OpenVMS AlphaServer 800 systems |
| FM-80BVN-60 | 60-month Node Software Supplemental Support for OpenVMS AlphaServer 800 systems |

Step 10a—Hardware and Software Supplemental Support Services (Europe only)

Europe does not have specific part numbers for Hardware and Software Supplemental Support Services. Prices can be quoted using the Excelerator tool; contact MCS Sales in your country for information on Hardware and Software Supplemental Support Services.

AlphaServer 800 System Diagram

1.	Three 32-bit PCI slots

2.	One 64-bit PCI/EISA slot

3.	Two EISA slots

4.	Parallel port

5.	Serial port

6.	Mouse port

7.	VGA port

8.	Keyboard port

9.	Remote console modem port

10.	Power inlet

11.	Serial port/terminal port (MMJ with data leads only)

12.	SCSI breakouts

AlphaServer 800 System Diagram

1.	Removable side cover of system enclosure

2.	Floppy diskette drive

3.	Removable media drives

4.	Lower and upper doors

5.	Control panel

6.	Hard disk drives

7.	System board

8.	CPU daughter card

9.	Power supply

Example:  AlphaServer 800 with KZPAC 3 port SCSI Controller using SCSI breakout for third port connection

AlphaServer 800 Pedestal System Specifications

| Shipping Dimension |  |  |  |  |
| --- | --- | --- | --- | --- |
| Height Width Depth Weight |  | 47 cm (18.5 in.) 60 cm (23.6 in) 77.5 cm (30.5 in) 33 kg (73 LB) typical 37 kg (82 LB) maximum |  |  |
| Installed Dimensions |  |  |  |  |
| Height Width Depth Weight |  | 45 cm (17.1 in.) 22.6 cm (8.9 in.) 65.8 cm (25.9 in.) 24.5 kg (54 LB) typical 28 kg (62 LB) maximum |  |  |
| Clearances | Operating |  |  | Service |
| Front Rear Left side Right side | 75 cm (29.5 in.) 15 cm (5.9 in.) None None |  |  | 75 cm (29.5 in.) 75 cm (29.5 in.) 75 cm (29.5 in.) 5 cm (2 in.) |
| Environmental |  |  |  |  |
| Temperature | Operating* Nonoperating Storage (60 days) Rate of change |  |  | 10-40° C (50-104° F) Not tested -40-66° C (-40-151° F) 11° C/hr (20° F/hr) |
| Relative humidity | Operating Nonoperating Storage (60 days) Rate of change |  |  | 20–80% 20–80% 10–95% 20%/hr |
| Maximum wet bulb temperature | Operating Storage (60 days) |  |  | 28° C (82° F) 46° C (115°F) |
| Minimum dew point temperature | Operating Storage (60 days) |  |  | 2° C (36° F) Not tested |
| Maximum heat dissipation | 380 Watt 1297 Btu/hr |  |  |  |
| Air flow and quality | Intake location Exhaust location Particle size Concentration |  |  | Front Rear N/A N/A |
| Altitude | Operating† Nonoperating |  |  | 3097 m (10,000 ft) 12190 m (40,000 ft) |
| Mechanical shock | Operating Nonoperating |  |  | 7.5 G 10 ms 20 G peak 30 ms |
| Vibration | Operating |  |  | 10-500 Hz .1 G peak |
| Acoustics | Operating |  |  | LNPEc (BELs) 6.0 MAX per ISO 7779 |
| Electrical—Power Supplies are universal 120/240 Vac |  |  |  |  |
| Nominal ac voltage Operating Voltage range Power source phase Nominal frequency Frequency range RMS current at nominal – –voltage (steady state) Maximum Power Consumption (Watts) | 100-120 Vac 90-128 Vac Single 60 Hz 59-61 Hz 7.0 Amps 380 Watts |  | 220-240 Vac 180-256 Vac Single 50 Hz 49-52 Hz 3.0 Amps |  |
| Power cord | Type Length U.S. plug |  | IEC 320 C13 190 cm (75 in.) NEMA 5-15 |  |

*	Maximum operating temperature at Sea Level. Reduce by 1 C (1.8 F) for each 600 m (2000 ft) above Sea Level.

† 	Higher altitudes are possible if maximum operating temperature is reduced (see Temperature, above); other restrictions may apply, such as maximum permissible altitude for hard drives.

AlphaServer 800 Pedestal System Specifications (continued)

| Regulatory |  |
| --- | --- |
| Agency approvals | UL Listed to UL1950 (3rd edition) CSA Certified to CAN/CSA-C22.2 No. 950-M95 TUV EN 60950 GS marked CB Certification pending FCC (Class B) Part 15, Subpart B (CFR 54,1995) EMC Directive 89/336/EEC EN 55022:1994 Class B EN 50082-1 1992 Light Industrial CE Class B VCCI Class II ITE AS/NZS 3548:1992 (Class B) |
| Reviewed to | AS 3260:1993 Australian/New Zealand Standard EN 60 950: 1995 European Norm IEC 950 |

AlphaServer 800 Rackmount System Specifications

| Shipping Dimension |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Same as Pedestal |  |  |  |  |  |
| Installed Dimensions |  |  |  |  |  |
| Height Width Depth Weight |  | 22 cm (8.6 in.) fits 8.75 in. [5U] standard EIA 310D standard EIA 310D (RETMA) 63.8 cm (25.1 in.) 24.5 kg (54 LB) typical 28 kg (62LB) maximum |  |  |  |
| Clearances | Service |  |  |  |  |
| Front Rear Sides | See requirements of specific cabinet Slides forward 68.6 cm (27 in.) then accessible from the top |  |  |  |  |
| Environmental |  |  |  |  |  |
| Temperature —Operating —Nonoperating —Storage (60 days) | 10-35° C (50-95° F) Not tested -40-66° C (-40-151° F) 11 c/hr (20 F/HR) |  |  |  |  |
| Relative humidity —Operating —Nonoperating —Storage (60 days) —Rate of change | 20–80% 20–80% 10–95% 20%/hr |  |  |  |  |
| Air flow and quality —Intake location —Exhaust location | Front Rear |  |  |  |  |
| Electrical—Power Supplies are universal 120/240 Vac |  |  |  |  |  |
| Same as Pedestal |  |  |  |  |  |
| Regulatory |  |  |  |  |  |
| Same as Pedestal |  |  |  |  |  |

Uninterruptable Power Supplies

Prestige 1250EXT

UPS offerings include EIA232 port for local or network monitoring and plug-in battery extension provisions to increase run time up to two hours.  For complete protection, UPS  products should be used with data line surge protectors and UPS monitoring software.

Solutions for Pedestal Systems

| 4N-AEABF-AA | Prestige 1250EXT, 1250 VA/900W, 1 phase, 50/60 Hz, 120V 6-foot cord with 5-15P plug, (4) 5-15R receptacle, 9 minutes battery at full UPS load. |
| --- | --- |
| 4N-AEABF-BF | Prestige 1250EXT, 1250 VA/900W, 1 phase, 50/60 Hz, 200-240V in/out selectable. Uses system power cord for detachable IEC320 input connection at UPS. Includes (3) IEC320 10A output receptacles, (2) output jumpers with IEC320 connectors to system. |
| 4N-AEABF-BB | Extension pack for 120V 60 Hz models, up to four packs at 30 minutes per pack supported |
| 4N-AEABF-BD | Extension pack for 240V 50 Hz models, up to four packs at 30 minutes per pack supported |
| 4N-AEAEO-PA | Hot-swap Power-Pass Module for 60 Hz UPS with (7) 5-15R Surge Protected outlets. |
| 4N-AEAEO-PB | Hot-swap Power-Pass Module for 50 Hz UPS with (6) IEC320, 10A surge protected outlets. |
| 4N-AEWAR-G1 | Prestige 5 year on-site exchange warranty upgrade (U.S. only). |

Rackmount UPS

| UPS Model | Application | Type | Recepticle module |
| --- | --- | --- | --- |
| 4N-AEAAH-FA 60Hz | Rackmount UPS, supports up to 5 systems | 3kVA Custom RM1 | Included2 |
| 4N-AEAAH-FB 50Hz | Rackmount UPS, supports up to 5 systems | 3kVA Custom RM1 | Included |
| 4N-AEAAJ-CL3 60Hz | Out of Cabinet UPS | 6kVA Prestige4 | Included |
| 4N-AEAAJ-CU 50Hz | Out of Cabinet UPS | 6kVA Prestige4 | Hardwired |

1.	Factory installation must be specified at time of order.  Up to two UPS systems, one UPS for up to 5 fully configured systems. Recommended UPS distribution option combines multiple UPS input plugs into a single L6-30P plug-in connection to simplify future UPS add-on.  Order 4NAEAAH-FF 60 Hz; 4N-AEAAH-FG 50 Hz).

2.	Includes six IEC receptacle strips for connection to IEC plugs in cabinet.

3.	208-120/208V shown; order 4N-AEAAJ-CT for 240-120/240V models

4.	Optional mobile stacker with siesmic mounting provision for 6kVA Prestige (4N-AEACH-HD)

| 4N-AEAAJ-CL | Prestige 6 kVA (4 kW), single phase, 60Hz, 208-120/208V, 6 ft. cord with L6-30P and (2) L6-30R, (8) 5-15R receptacle. Modular hot-swap design with 7 minute battery at full UPS load, extendible plug and play batteries and receptacle provisions. Substitute -CT for 240V 240/120V operation. |
| --- | --- |
| 4N-AEAAJ-CU | Prestige 6 kVA (4 kW) 50Hz package, single phase, 50Hz, 200-240V in/out selectable, hardwired input/output. |
| 4N-AEWAR-G3/G4 | Prestige 5 year on-site exchange warranty upgrade for 4N-AEAAH-CM/CT. G4 provides on-site remedial and start-up service. |

UPS Monitoring and Unattended Shutdown Software (for above UPS systems only) is included in ServerWorks Manager Kits shipping with all Alphaservers.  Order Cable Kit separately.

| 4N-ONLIN-NT | Cable Kit for Windows NT and DIGITAL UNIX |
| --- | --- |
| 4N-ONLIS-FE | Cable Kit for OpenVMS |

Network Management or multi system shutdown (via SNMP or ServerWORKS Manager) for DIGITAL UNIX and OpenVMS requires Network Adapter option.

| 60 Hz Applications | 50 Hz Application | Network Adapter Option |
| --- | --- | --- |
| 4N-AEAEO-DB/DD | 4N-AEAEO-DB/DD | Prestige 3 kVA RM, DB=Twisted pair; DD=ThinWire |
| 4N-AEAEO-DA/DC | 4N-AEAEO-DB/DD | Prestige 6 kVA and PUPS Plus UPS, DA/DB=Twisted pair; DC=ThinWire |

Note:  Multiple system shutdown via OpenVMS requires multi-port hardware option (RN-JMIU4-AB).  Ports can be daisy chained in increments of four.